PC6—TD4-SNR

This is a bid for a dual display universal digital programmable temperature controller that supports many type of thermocouple sensor. It can be powered by 90-265V AC/DC or 18-30V AC/DC or 24V DC, Comsumption5VA. DIN 1/16.It can also worked as simple digital adjustor.

Manual/ Auto-tuning PID adjustment configurable by software

Manual PID adjustment: set your desire output value to suit your application

Auto-tuning PID adjustment: 3 build-in Auto-Tuning Time proportion PID Algorithms to choose for different applications or environments to be controlled. (for variety of industries or home projects heating or cooling applications.) 

Specifications

Display: Dual display for Fahrenheit(F) and Celsius(C)

Range: -1999 to 9999 (depends on the input signal)

Main Output: SSR: Open circuit: 4V, Closed Circuit: 24V 40mA DC

Manual/Auto-tuning PID cooling/heating control( or normal alarm output)

1 RELAY alarm:Normal open,  capacity 250V/3A AC or 30V/3A DC

7 different Dual Output combinations with: 
Accuracy: 0.2% 
Input: Thermo Resistor: PT100, Cu50

Thermocouple: J,S,K,E,Wre3 ~ Wre25 

Dimension: DIN: 1/16 (48mmX48mmX80mm)

Weight: about 150g

Individually programmable PID control parameters. 

P, I, d, controlling period, digital filter coefficient, and more… (for control theory experts only)
SSR control ready to connect external Solid State Relay to handle High current. Applications.

Compact design

DIN 1/16, great form-factor to be included/build-in to your product.

Apply with step control, 1 independent PID control for heating/cooling over temperature,pressure,level,flowm,etc..Widely used in auto system in line of light industry, chemistry, machine, metallurgy,ceramics,pertrification industry, or temperature control and adjust system of food & beverage; oven; furnance, plastic extruder heating process etc;

One key operation, Auto-tuning PID/ Fuzzy PID control.Easy operation,Reliable performance

Input: TC(K、J、S、E)/ RTD（Pt100、CU50）universal input, or 4-20mA/0-10V、0-75mV signal from standard pressure/level transmitter,

Control output: RELAY  SSR  SCR  4-20mA  0-10V (If 4-20mA or 0-10V DC output,you can choose PID control or analog output)

Alarm output: 1 line output,7 kinds of alarm mode: high / low  / high deviation / low deviation/ interval / out of interval /breakage alarm
PC7—TA4-SNR+K thermocouple

This is a bid for a dual display universal digital programmable temperature controller that supports many type of thermocouple sensor. It can be powered by 90-265V AC/DC or 18-30V AC/DC or 24V DC, Comsumption5VA. DIN 1/16

3 build-in Auto-Tuning Time proportion PID Algorithms to choose for different applications or environments to be controlled. (for variety of industries or home projects heating or cooling applications.) 

Specifications

Display: Dual display for Fahrenheit(F) and Celsius(C)

Range: -1999 to 9999 (depends on the input signal)

Main Output: SSR: Open circuit: 4V, Closed Circuit: 24V 40mA DC

Auto-tuning PID cooling/heating control

1 RELAY alarm:Normal open,  capacity 250V/3A AC or 30V/3A DC

7 different Dual Output combinations with: 
Accuracy: 0.2% 
Input: Thermo Resistor: PT100, Cu50

Thermocouple: J,S,K,E,Wre3 ~ Wre25 

Dimension: DIN: 1/16 (48mmX48mmX80mm)
Net Weight: about 150g

Individually programmable PID control parameters. 

P, I, d, controlling period, digital filter coefficient, and more… (for control theory experts only)
SSR control ready to connect external Solid State Relay to handle High current. Applications.

K type thermocouple, 0-1320℃, 1m cable(3 feet).

Compact design

DIN: 1/16, great form-factor to be included/build-in to your product.

Apply with step control, 1 independent PID control for heating/cooling over temperature,pressure,level,flowm,etc..Widely used in auto system in line of light industry, chemistry, machine, metallurgy,ceramics,pertrification industry, or temperature control and adjust system of food & beverage; oven; furnance, plastic extruder heating process etc;

One key operation, Auto-tuning PID/ Fuzzy PID control.Easy operation,Reliable performance

Input: TC(K、J、S、E)/ RTD（Pt100、CU50）universal input, or 4-20mA/0-10V/0-75mV signal from standard pressure/level transmitter,

Control output: RELAY  SSR  SCR  4-20mA  0-10V (If 4-20mA or 0-10V DC output,you can choose PID control or analog output)

Alarm output: 1 line output,7 kinds of alarm mode: high / low  / high deviation / low deviation/ interval / out of interval /breakage alarm
PC8—TA8-SNR+3 feet K thermocouple

This is a bid for a dual display universal digital programmable temperature controller that supports many type of thermocouple sensor. It can be powered by 90-265V AC/DC or 18-30V AC/DC or 24V DC, Comsumption5VA. DIN 1/8.

3 build-in Auto-Tuning Time proportion PID Algorithms to choose for different applications or environments to be controlled. (for variety of industries or home projects heating or cooling applications.) 

Specifications

Display: Dual display for Fahrenheit(F) and Celsius(C)

Range: -1999 to 9999 (depends on the input signal)

Main Output: SSR: Open circuit: 4V, Closed Circuit: 24V 40mA DC

Auto-tuning PID cooling/heating control

1 RELAY alarm:Normal open,  capacity 250V/3A AC or 30V/3A DC

7 different Dual Output combinations with: 
Accuracy: 0.2% 
Input: Thermo Resistor: PT100, Cu50

Thermocouple: J,S,K,E,Wre3 ~ Wre25 

Dimension:DIN 1/8 ( 48HX96WX100Lmm )

Net Weight: about 170g

Individually programmable PID control parameters. 

P, I, d, controlling period, digital filter coefficient, and more… (for control theory experts only)
SSR control ready to connect external Solid State Relay to handle High current. Applications.

K type thermocouple, 0-1320℃, 1m cable(3 feet).

Compact design

DIN 1/8 ( 48HX96WX100Lmm ) size, great form-factor to be included/build-in to your product.

Apply with step control, 1 independent PID control for heating/cooling over temperature,pressure,level,flowm,etc..Widely used in auto system in line of light industry, chemistry, machine, metallurgy,ceramics,pertrification industry, or temperature control and adjust system of food & beverage; oven; furnance, plastic extruder heating process etc;

One key operation, Auto-tuning PID/ Fuzzy PID control.Easy operation,Reliable performance

Input: TC(K、J、S、E)/ RTD（Pt100、CU50）universal input, or 4-20mA/0-10V/0-75mV signal from standard pressure/level transmitter,

Control output: RELAY  SSR  SCR  4-20mA  0-10V (If 4-20mA or 0-10V DC output,you can choose PID control or analog output)

Alarm output: 1 line output,7 kinds of alarm mode: high / low  / high deviation / low deviation/ interval / out of interval /breakage alarm
PC9—TA7-SNR

This is a bid for a dual display universal digital programmable temperature controller that supports many type of thermocouple sensor. It can be powered by 90-265V AC/DC or 18-30V AC/DC or 24V DC, Comsumption5VA. Size:72W*72H*100L mm

3 build-in Auto-Tuning Time proportion PID Algorithms to choose for different applications or environments to be controlled. (for variety of industries or home projects heating or cooling applications.) 

Specifications

Display: Dual display for Fahrenheit(F) and Celsius(C)

Range: -1999 to 9999 (depends on the input signal)

Main Output: SSR: Open circuit: 4V, Closed Circuit: 24V 40mA DC

Auto-tuning PID cooling/heating control

1 RELAY alarm:Normal open,  capacity 250V/3A AC or 30V/3A DC

7 different Dual Output combinations with: 
Accuracy: 0.2% 
Input: Thermo Resistor: PT100, Cu50

Thermocouple: J,S,K,E,Wre3 ~ Wre25 

Dimension: 72mmX72mmX100mm 

Net Weight: about 200g

Individually programmable PID control parameters. 

P, I, d, controlling period, digital filter coefficient, and more… (for control theory experts only)
SSR control ready to connect external Solid State Relay to handle High current. Applications.

Compact design

72W*72H*100L mm size, great form-factor to be included/build-in to your product.

Apply with step control, 1 independent PID control for heating/cooling over temperature,pressure,level,flowm,etc..Widely used in auto system in line of light industry, chemistry, machine, metallurgy,ceramics,pertrification industry, or temperature control and adjust system of food & beverage; oven; furnance, plastic extruder heating process etc;

One key operation, Auto-tuning PID/ Fuzzy PID control.Easy operation,Reliable performance

Input: TC(K、J、S、E)/ RTD（Pt100、CU50）universal input, or 4-20mA/0-10V/0-75mV signal from standard pressure/level transmitter,

Control output: RELAY  SSR  SCR  4-20mA  0-10V (If 4-20mA or 0-10V DC output,you can choose PID control or analog output)

Alarm output: 1 line output,7 kinds of alarm mode: high / low  / high deviation / low deviation/ interval / out of interval /breakage alarm
PC10—TA6-SNR

This is a bid for a dual display universal digital programmable temperature controller that supports many type of thermocouple sensor. It can be powered by 90-265V AC/DC or 18-30V AC/DC or 24V DC, Comsumption5VA. Size:48W*96H*80L mm

3 build-in Auto-Tuning Time proportion PID Algorithms to choose for different applications or environments to be controlled. (for variety of industries or home projects heating or cooling applications.) 

Specifications

Display: Dual display for Fahrenheit(F) and Celsius(C)

Range: -1999 to 9999 (depends on the input signal)

Main Output: SSR: Open circuit: 4V, Closed Circuit: 24V 40mA DC

Auto-tuning PID cooling/heating control

1 RELAY alarm:Normal open,  capacity 250V/3A AC or 30V/3A DC

7 different Dual Output combinations with: 
Accuracy: 0.2% 
Input: Thermo Resistor: PT100, Cu50

Thermocouple: J,B,S,K,E,Wre3 ~ Wre25 

Dimension: 48WX96HX80Lmm 

Net Weight: about 170g

Individually programmable PID control parameters. 

P, I, d, controlling period, digital filter coefficient, and more… (for control theory experts only)
SSR control ready to connect external Solid State Relay to handle High current. Applications.

Compact design

72W*72H*100L mm size, great form-factor to be included/build-in to your product.

Apply with step control, 1 independent PID control for heating/cooling over temperature,pressure,level,flowm,etc..Widely used in auto system in line of light industry, chemistry, machine, metallurgy,ceramics,pertrification industry, or temperature control and adjust system of food & beverage; oven; furnance, plastic extruder heating process etc;

One key operation, Auto-tuning PID/ Fuzzy PID control.Easy operation,Reliable performance

Input: TC(K、J、S、E)/ RTD（Pt100、CU50）universal input, or 4-20mA/0-10V/0-75mV signal from standard pressure/level transmitter,

Control output: RELAY  SSR  SCR  4-20mA  0-10V (If 4-20mA or 0-10V DC output,you can choose PID control or analog output)

Alarm output: 1 line output,7 kinds of alarm mode: high / low  / high deviation / low deviation/ interval / out of interval /breakage alarm
