# Upto 70m  HDBaseT HDMI Extender over Single CAT5E/6/7 Cable with Bi-directional IR Remote Support 4Kx2K, 3D, 1080P Deep Colour Ultra HD

Description:

This HDbaseT extender is based on HDbaseT technology, it can extend your high resolution up to 4Kx2K HDMI display up to 70 meters away your HDMI source by using single Cat5e/6/7 cable. Perfect for home cinema or professional AV installations, The HDBaseT Extender is also able to extend infrared signals, allowing AV equipment to be controlled up to 70m away. An integrated Ethernet hub also allows you to conveniently connect PC/MAC,xbox and other network equipment over the same single CAT 5e/6/7 cable.

Features

1. Supports HDMI resolution up to 4K x 2K

2. Supports uncompressed HDMI signal

3. Supports uncompressed audio such as LPCM

4. Supports copressed audio such as DTS Digital and Dolby Digital

5. Supports CEC pass through

6. Supports 3D pass through

7. With Built-in IR extender function

8. Cat5e/6/7 cable follows the standard of IEEE-568B

9. Supports 5 play

Specifications:

HDMI signal: HDMI1.4, compatible with HDCP; support CEC and 24bits deep color 
Resolution support: 480i, 480p, 576i, 576p, 720p, 1080i, 1080p, 3D, 4kx2k 
Ethernet cable support: CAT5E, CAT6, CAT6A, CAT7 
IR: Support 20~60kHz wide frequency devices and Bi-directional IR transmission 
Support audio format: uncompressed audio output such as LPCM, and compressed audio output like DTS-HD, Dolby True HD

Package Content: 
HDMI to HDBaseT Sender x1 
HDBaseT to HDMI Receiver x1 
IR blaster cable x1 
IR receiver cable x1 
DC12V 2A Power Adapter x1 
User Manual x1

Note:

1.Transmission distance:

CAT6A/7: 230ft/70m for up to 1080P@60Hz

CAT5E/6: 196ft/60m for up to 1080P@60Hz

CAT6/7:131ft/40m for 4kx2k 30Hz

2.Only need to use a power supply，the proposed power supply is used in the RX side