TENVIS JPT3815W wireless Webcam iPhone Android IP camera Audio Video WIFI Camera OSD IR Motion Detction

Features:

1. Unique and elegant design, High sensitivity sensor CMOS

2. Support smart mobile remote surveillance anywhere anytime

3. PT control (Horizontal angle 270 degree; Vertical angle 90degree)

4. Adopt optimized MJPEG video compression format

5. Support Motion detection alarm，support image snapshot

6. Motion detection alert via email or upload image to FTP

7.12 Meters Night Vision

8.9 Channels in the same time

9.Free 36 Channels Client Software

10.WIFI/Wired Both

12.Speak and Hear--2 way Audio

Specifications:

| Camera | Image sensor | 1/4 Color CMOS |
| --- | --- | --- |
|  | Lens | 3.6mm |
|  | Infrared LED | 10 pcs ø5 |
|  | Pixel | 300,000 |
|  | Night vision Distance | 12m |
|  | AWS/AGC/AES/ exposure | Auto |
|  | Min. Illumination | 0.1Lux |
| Audio | Audio | 2- way audio |
|  | Input | Built-in Microphone |
|  | Output | Built-in Speaker |
| Video | Image format | PAL/NTSC |
|  | Compression | MJPEG |
|  | Max. frame rate | 25 fps |
|  | Resolution | 640×480(VGA), 320×240(QVGA) |
| Pan/Tilt | PT Angle | Horizontal 270°; Vertical 90° |
|  | Level Speed | 0 - 16 °/s |
|  | Vertical Speed | 0 - 16 °/s |
| Network | Wi-fi | IEEE 802.11b/g |
|  | Network Interface | RJ-45 (10BASE-T/100BASE-TX) |
|  | Supporting Protocols | TCP/UDP/IP/ARP/ICMP/DHCP/DNS/HTTP/ FTP/SMTP/NTP/PPPOE/UPNP/DDNS |
| Alarm | Motion detection | One configurable zone |
|  | Alarm events | • Notification via email, FTP • Video recording to local storage |
|  | Periodic sending | Send pictures to email/FTP within theprovided time |
| General | User Authentication | Authentication: user/password; administrator/operator/general user |
|  | Web Browsers | IE 6.0 or above version, Mozilla Firefox, Safari, Opera, Chrome ,etc |
|  | Simultaneous Viewers | VGA (640x480) resolution support 4Simultaneous viewers, QVGA (320x240) resolution support 10Simultaneous viewers |
|  | Mobile View | Support Nokia, Android, Windows mobile phone, Iphone and other smart mobile phones |
|  | Power | DC 5V 2A |
|  | Power Consumption | 4W/6W (Infrared ON/OFF) |
|  | Operating Temperature | -10 to +55 °C |
|  | Storage Temperature | -20 to +60 °C |
|  | Operating Humidity | 20- 80%RH |
|  | Storage Humidity | 20- 95%RH |
|  | Package Dimension | 215mm x135mm x185mm (L x W x H) |
|  | Color | Black |
|  | Gross Weight | 510g |
| Certificates |  | CE; FCC; RoHS |

One Set Included:

- 1 x Wireless IP Camera

- 1 x Wi-Fi Antenna

- 1 x Mounting Bracket

- 1 x Power Adapter (US plug)

- 1 x Software CD

- 1 x User Manual

- Standard Factory Package