“O” Scale






Kit 307-A
3 Bay Trolley Barn
Extension Kit Instructions
Note 1:  For ease of assembly, it is recommended that the Extension Kit #307-A be added and assembled along with Kit #307.  In this case, follow the instruction for Kit #307 and add the additional frame and the 3rd sidewalls and roofs provided in Kit 307-A, when called for in the Kit #307 instructions.

Note 2:  If adding Kit #307-A to an already completed Kit #307, you must first separate one of the end walls, along with the end frame, from the sidewalls and roof before assembly can begin.
Pre-Assembly Instructions
1. Before beginning assembly, we recommend that you familiarize yourself with the parts by reviewing the parts list and reading through the instructions.
2. If desired, perform any detail painting prior to assembly.

3. It is recommended that this kit be mounted on a 1/4” or 3/8” plywood or Masonite base for added stability (base not included with kit).
4. When gluing wall section together, we recommend that you first tack glue the pieces at the corners, inspect the alignment of the parts, and when satisfied with the alignment, run a second bead of glue to fill any gaps in the joint.

Assembly Instructions
1. Begin by adding one of the new sidewalls to each of the two completed sidewalls.  Inspect the contact edges and file the edges to square up the bonding surface.  Glue the new wall sections to the existing wall sections and allow them to dry.  
2. Once the sidewall sections have dried, glue in the windows from the back side of the wall sections.
3. While the glue dries on the sidewalls, prepare to assemble the additional wooden building frames.  Layout the framing template on a flat surface and again cover with wax paper.  Dry fit the wooden pieces to make sure they are clean and flush prior to gluing in place.  Assemble and glue the additional wooden frame.

4. When sidewalls and frame are dry, re-install the removed end wall section to the sidewalls, again filing and squaring up the contact surfaces, lining up the stone patterns of the wall sections and then glue in place.  Use a small square to help keep the walls at 90 degrees to each other. 
5. Once the extended building shell is complete and dry, glue in the new wooden frame at the joint where the new sidewalls were added.  Use a roof section to help in the placement of the middle frame.  The roof section should land at the centerline of the frame. See illustration below:

[image: image2.jpg]





Illustration 1.
6. Next, dry fit and glue in place the roof sections.  
7. If you desire to use the roof gravel, mask off all exposed wall areas and then apply a spray adhesive to the roof top.  Sprinkle roof with gravel.  When dry, shake off excess and remove the tape masking.

Congratulation!  You have completed your KorModels kit and we hope that it provides you many years of enjoyment on your layout.
Roof at Centerline of Frame.








[image: image1]