FORD KugA Specifications

ENGINES

Diesel

|  |  |  |  |  | 1.5-litre TDCi (120 PS) | 2.0-litre TDCi (150 PS) | 2.0-litre TDCi (180 PS) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  |  |  |  | Inline four-cylinder turbo diesel, transverse | Inline four-cylinder turbo diesel, transverse | Inline four-cylinder turbo diesel, transverse |
| Displacement | cm³ |  |  |  | 1499 | 1997 | 1997 |
| Bore | mm |  |  |  | 73.5 | 85.0 | 85.0 |
| Stroke | mm |  |  |  | 88.3 | 88.0 | 88.0 |
| Compression ratio |  |  |  |  | 16:1 | 16:1 | 16:1 |
| Max power | kW (PS) |  |  |  | 88 (120) | 110 (150) | 132 (180) |
|  | at rpm |  |  |  | 3600 | 3500 | 3500 |
| Max torque | Nm |  |  |  | 270/300* | 370 | 400 |
|  | at rpm |  |  |  | 1500 - 2000 | 2000 - 2500 | 2000 - 2500 |
| Valve gear |  |  |  |  | SOHC with 2 valves per cylinder | DOHC with 4 valves per cylinder | DOHC with 4 valves per cylinder |
| Cylinders |  |  |  |  | 4 in line | 4 in line | 4 in line |
| Camshaft drive |  |  |  |  | Timing belt (crankshaft to intake) with dynamic tensioner; Intake to exhaust chain with hydraulic tensioner | Belt-driven cams with primary drive tensioner | Belt-driven cams with primary drive tensioner |
| Crankshaft bearings |  |  |  |  | Steel, 8 counterweights, 5 main bearings | Steel, 8 counterweights, 5 main bearings | Steel, 8 counterweights, 5 main bearings |
| Engine management |  |  |  |  | Bosch | Delphi | Delphi |
| Fuel injection |  |  |  |  | Common-Rail direct injection, turbocharger | Common-Rail direct injection, turbocharger | Common-Rail direct injection, turbocharger |
| Emission control |  |  |  |  | Oxidation catalyst with water-cooled EGR and closed loop DPF | Oxidation catalyst with water-cooled EGR and closed loop DPF | Oxidation catalyst with water-cooled EGR and closed loop DPF |
| Emission level |  |  |  |  | Euro 6 | Euro 6 | Euro 6 |
| Lubrication system |  |  |  |  | Pressure-fed lubrication system with full flow oil filter and oil cooler | Pressure-fed lubrication system with full flow oil filter and oil cooler | Pressure-fed lubrication system with full flow oil filter and oil cooler |
| Lubrication system capacity with filter |  |  | L |  | 4.6 | 5.7 | 5.7 |
| Cooling system |  |  |  |  | Water pump with thermostat, valves and vent | Water pump with thermostat, valves and vent | Water pump with thermostat, valves and vent |
| Cooling system capacity, incl. heater |  |  |  | L | 8.5 | 8.5 | 8.5 |

| * with 6-speed PowerShift Petrol |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1.5-litre EcoBoost (120 PS) |  | 1.5-litre EcoBoost (150 PS) | 1.5-litre EcoBoost (182 PS) |
| Type |  |  |  |  | Inline four-cylinder turbo petrol, transverse |  | Inline four-cylinder turbo petrol, transverse | Inline four-cylinder turbo petrol, transverse |
| Displacement | cm³ |  |  |  | 1498 |  | 1498 | 1498 |
| Bore | mm |  |  |  | 79.0 |  | 79.0 | 79.0 |
| Stroke | mm |  |  |  | 76.4 |  | 76.4 | 76.4 |
| Compression ratio |  |  |  |  | 10.0:1 |  | 10.0:1 | 10.0:1 |
| Max power | kW (PS) |  |  |  | 88 (120) |  | 110 (150) | 134 (182) |
|  | at rpm |  |  |  | 5500 |  | 6000 | 6000 |
| Max torque | Nm |  |  |  | 240 |  | 240 | 240 |
|  | at rpm |  |  |  | 1600 - 3000 |  | 1600 - 4000 | 1600 - 5000 |
| Valve gear |  |  |  |  | DOHC with 4 valves per cylinder, twin independent variable cam timing |  | DOHC with 4 valves per cylinder, twin independent variable cam timing | DOHC with 4 valves per cylinder, twin independent variable cam timing |
| Cylinders |  |  |  |  | 4 in-line |  | 4 in-line | 4 in-line |
| Camshaft drive |  |  |  |  | Low friction Belt-in-Oil with dynamic tensioner |  | Low friction Belt-in-Oil with dynamic tensioner | Low friction Belt-in-Oil with dynamic tensioner |
| Crankshaft bearings |  |  |  |  | Cast iron, 4 counterweights, 5 main bearings |  | Cast iron, 4 counterweights, 5 main bearings | Cast iron, 4 counterweights, 5 main bearings |
| Engine management |  |  |  |  | MED17 with CAN-Bus and individual cylinder knock control. FGEC Software |  | MED17 with CAN-Bus and individual cylinder knock control. FGEC Software | MED17 with CAN-Bus and individual cylinder knock control. FGEC Software |
| Fuel injection |  |  |  |  | Common Rail/Direct Injection |  | Common Rail/Direct Injection | Common Rail/Direct Injection |
| Emission level |  |  |  |  | Euro 6 |  | Euro 6 | Euro 6 |
| Lubrication system |  |  |  |  | Electronically controlled variable displacement oil pump |  | Electronically controlled variable displacement oil pump | Electronically controlled variable displacement oil pump |
| Lubrication system capacity with filter |  |  | L |  | 3.8 |  | 3.8 | 3.8 |
| Cooling system |  |  |  |  | Water pump with thermostat, valves and vent |  | Water pump with thermostat, valves and vent | Water pump with thermostat, valves and vent |
| Cooling system capacity, including heater |  |  |  | L | 6.3 |  | 6.3 | 6.3 |

### DRIVE LINE

| Drive options: |  | AWD: Electronically controlled Ford Intelligent All Wheel Drive with variable torque distribution |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FWD: Front-wheel drive |  |  |  |  |  |  |
| Transmission: |  | 6-speed manual transmission 6-speed PowerShift automatic transmission 6-speed torque converter automatic transmission |  |  |  |  |  |  |
| Ratios |  | 1.5-litre TDCi, FWD (120 PS) Manual | 1.5-litre TDCi, FWD (120 PS) PowerShift | 2.0-litre TDCi, FWD (150 PS) Manual |  |  | 2.0-litre TDCi, AWD (150 PS) Manual |  | 2.0-litre TDCi, AWD (150 PS) PowerShift |  |
|  | 1. Gear | 3.583 | 3.583 | 3.583 |  |  | 3.583 |  | 3.583 |  |
|  | 2. Gear | 1.864 | 1.952 | 1.864 |  |  | 1.864 |  | 1.952 |  |
|  | 3. Gear | 1.194 | 1.194 | 1.194 |  |  | 1.156 |  | 1.194 |  |
|  | 4. Gear | 0.868 | 0.829 | 0.868 |  |  | 0.816 |  | 0.829 |  |
|  | 5. Gear | 0.943 | 0.943 | 0.943 |  |  | 0.886 |  | 0.943 |  |
|  | 6. Gear | 0.789 | 0.756 | 0.789 |  |  | 0.737 |  | 0.756 |  |
|  | Reverse gear | 1.423 | 4.843 | 1.423 |  |  | 1.423 |  | 4.843 |  |
|  | Final drive, 1 – 4 5, 6, R | 4.063 2.995 | 4.533 3.091 | 4.063 2.995 |  |  | 4.533 3.238 |  | 4.533 3.091 |  |
| Ratios |  | 2.0-litre TDCi, AWD (180 PS) Manual | 2.0-litre TDCi, AWD (180 PS) PowerShift | 1.5-litre, EcoBoost FWD (120 PS) Manual |  |  | 1.5-litre EcoBoost FWD (150 PS) Manual |  |  | 1.5-litre EcoBoost, AWD (182 PS) Auto |
|  | 1. Gear | 3.583 | 3.583 | 3.818 |  |  | 3.818 |  |  | 4.584 |
|  | 2. Gear | 1.864 | 1.952 | 2.150 |  |  | 2.150 |  |  | 2.964 |
|  | 3. Gear | 1.156 | 1.194 | 1.423 |  |  | 1.423 |  |  | 1.912 |
|  | 4. Gear | 0.816 | 0.829 | 1.029 |  |  | 1.029 |  |  | 1.446 |
|  | 5. Gear | 0.889 | 0.943 | 1.129 |  |  | 1.129 |  |  | 1.000 |
|  | 6. Gear | 0.737 | 0.756 | 0.943 |  |  | 0.943 |  |  | 0.746 |
|  | Reverse gear | 1.423 | 4.843 | 5.433 |  |  | 1.423 |  |  | 2.943 |
|  | Final drive, 1 – 4 5, 6, R | 4.533 3.238 | 4.533 3.091 | 4.063 2.955 |  |  | 4.063 2.955 |  |  | 3.51 |

### BODY

| ### BODY STRUCTURE | Computer-optimised, high-efficiency, unitary-welded steel body with plastic front wings incorporating rigid occupant cell and front and rear energy-absorbing crumple zones; direct-glazed windshield. |
| --- | --- |
| Anti-corrosion | Multi-stage paint and body protection process, including extensive pre-coating of zinc on steel panels, phosphate coat, electro coat primer, primer/surfacer, basecoat/clear coat topcoat system, comprehensive wax injection of cavities, PVC under body coating plus stone chip protection. Particular attention paid to sealing flanges with rust-inhibiting adhesive and thick PVC sealing beads. Front plastic wheel arch liners, rear textile wheel arch liners, plastic anti-scuff panels on rear load sill. |

CHASSIS

| Front axle: | Independent MacPherson struts with offset coil spring over gas filled damper units and lower L-Arms with optimised front rubber bushings and rear hydro-bush mounted on separate reinforced cross-member sub-frame, anti-roll bar. Dual path top mounts. |
| --- | --- |
| Rear axle: | Fully independent multi-link rear axle with control blade with large dampers and rebound springs. Anti-roll bar mounted to knuckle with double ball-joint link. |
| Steering: | Electric Power Assisted Steering (EPAS) as standard Turns lock-to-lock: 2.6 |

| Turning circle: | 11.1m |  |
| --- | --- | --- |
| Braking: | Dual circuit, diagonally split, hydraulically operated front and rear with disc brakes. Vacuum servo assisted with electronically controlled four-channel brake distribution ABS and optimised brake assist, ESP with Ford Trailer Sway Control and Rollover mitigation offered |  |
| Dimensions: | Front discs (ventilated): | 300mm x 25mm 320mm x 25mm |
|  | Rear discs (solid): | 280mm x 11mm |

WHEELS & TYRES

|  | Wheels | Tyres |
| --- | --- | --- |
| Standard | 7.5 J x 18-inch alloy | 235/50 R18 |
| Optional | 8 J x 19-inch alloy | 235/40 RF19 |
| Spare wheel | 17-inch with T155/70 R17 tyre (optional; instant mobility kit as standard) |  |

DIMENSIONS

|  |  | Kuga | Kuga Vignale and ST-Line |
| --- | --- | --- | --- |
| ##### Overall length | mm | 4531 | 4541 |
| Overall width with mirrors | mm | 2086 | 2086 |
| Overall width without mirrors | mm | 1838 | 1856 |
| Overall width with mirrors folded | mm | 1911 | 1911 |
| Overall height with roof rails | mm | 1703/1694* | 1703/1694* |
| Wheelbase | mm | 2690 | 2690 |
| ##### Track front | mm | 1573/1570* | 1571 |
| ##### Track rear | mm | 1583 | 1583 |
| Shoulder room first row | mm | 1421 | 1418 |
| ##### Shoulder room second row | mm | 1398 | 1398 |
| ##### Headroom with sunroof first row | mm | 973 | 973 |
| ##### Headroom without sunroof ##### first row |  | 1013 | 1013 |
| Headroom with sunroof ##### second row | mm | 950 | 950 |
| Headroom without sunroof second row |  | 990 | 990 |
| Legroom first row | mm | 1096 | 1096 |
| ##### Legroom second row | mm | 934 | 934 |
| Cargo volume behind first row (2-seat mode; laden to roof) with full size spare wheel (if applicable) | L | 1603 | 1603 |
| Closed cargo compartment second row (under cargo shade) with mini spare wheel (if applicable) | L | 456 | 456 |
| Loading length at floor to first row | mm | 1696 | 1696 |
| ##### Loading length at floor to second row with adjustable load floor min/max | mm | 844/871 | 844/871 |
| ##### Load width between wheel arches | mm | 1031 | 1031 |
| Load opening width at floor | mm | 1081 | 1081 |
| ##### Load opening height (at vehicle centreline) | mm | 809 | 809 |
| ##### Cargo Height (at rear wheel centerline) with adjustable load floor min/max | mm | 895/1002 | 895/1002 |
| ##### Lift-over Height (curb) with adjustable load floor min/max | mm | 719/721* | 721/707* |

*With Ford Intelligent All Wheel Drive

WEIGHTS

|  | Kerb weight (kg)# | Gross Vehicle Mass (kg) | Gross Train Mass (kg) | Max. Towable Mass (braked) (kg) | Max. Towable Mass (unbraked) (kg) | Max. Roof Load (kg) |
| --- | --- | --- | --- | --- | --- | --- |
| 1.5l TDCi (120 PS) 6-speed manual FWD | 1591 | 2100 | 3300 | 1200 | 750 | 75 |
| 1.5l TDCi (120 PS) 6-speed PowerShift FWD | 1605 | 2100 | 3300 | 1200 | 750 | 75 |
| 2.0l TDCi (150 PS) 6-speed manual FWD | 1614 | 2250 | 4150 | 1900 | 750 | 75 |
| 2.0l TDCi (150 PS) 6-speed manual AWD | 1702 | 2250 | 4350 | 2100 | 750 | 75 |
| 2.0l TDCi (150 PS) 6-speed PowerShift AWD | 1716 | 2230 | 4330 | 2100 | 750 | 75 |
| 2.0l TDCi (180 PS) 6-speed manual AWD | 1702 | 2250 | 4350 | 2100 | 750 | 75 |
| 2.0l TDCi (180 PS) 6-speed PowerShift AWD | 1716 | 2230 | 4330 | 2100 | 750 | 75 |
| 1.5l EcoBoost (120 PS) 6-speed manual FWD | 1579 | 2100 | 3900 | 1800 | 750 | 75 |
| 1.5l EcoBoost (150 PS) 6-speed manual FWD | 1579 | 2100 | 4100 | 2000 | 750 | 75 |
| 1.5l EcoBoost (182 PS) 6-speed auto AWD | 1686 | 2200 | 4050 | 1850 | 750 | 75 |

# Represents the lightest kerb weight assuming driver at 75 kg, full fluid levels and 90% fuel levels, subject to manufacturing tolerances and options, etc., fitted. Towing limits quoted represent the maximum towing ability of the vehicle at its Gross Vehicle Mass to restart on a 12 per cent gradient at sea level. The performance and economy of all models will be reduced when used for towing. Nose weight limit is a maximum of 50 kg on all models.  Gross Train Mass includes trailer weight

PERFORMANCE, FUEL ECONOMY, EMISSIONS

|  | Power | Max speed | 0-100 km/h 0-62 mph Ø | 50-100 km/h 31-62 mph Ø* | Fuel consumption l/100 km (mpg) ØØ |  |  | CO2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PS | km/h (mph) | sec | sec | Urban | Extra Urban | Comb. | g/km |
| 1.5l TDCi, FWD 6-speed manual | 120 | 173 (107) | 12.7 | 10.9 | 4.8 (58.8) | 4.2 (67.2) | 4.4 (64.2) | 115 |
| 1.5l TDCi, FWD 6-speed PowerShift | 120 | 171 (106) | 12.4 | N/A | 5.1 (55.4) | 4.6 (61.4) | 4.8 (58.8) | 124 |
| 2.0l TDCi, FWD 6-speed manual | 150 | 194 (121) | 10.1 | 9.1 | 5.4 (52.2) | 4.3 (65.7) | 4.7 (60.1) | 122 |
| 2.0l TDCi, AWD 6-speed manual | 150 | 192 (119) | 9.9 | 9.3 | 6.0 (47.1) | 4.7 (60.1) | 5.2 (54.3) | 135 |
| 2.0l TDCi, AWD 6-speed PowerShift | 150 | 190 (118) | 10.9 | N/A | 5.5 (51.4) | 4.9 (57.6) | 5. 2 (54.3) | 134 |
| 2.0l TDCi, AWD 6-speed manual | 180 | 202 (126) | 9.2 | 8.1 | 6.0 (47.1) | 4.7 (60.1) | 5.2 (54.3) | 135 |
| 2.0l TDCi, AWD 6-speed PowerShift | 180 | 200 (124) | 10.0 | N/A | 5.5 (51.4) | 4.9 (57.6) | 5.2 (54.3) | 134 |
| 1.5l EcoBoost, FWD 6-speed manual | 120 | 180 (112) | 12.5 | ? | 7.9 (35.7) | 5.4 (52.3) | 6.3 (44.8) | 145 |
| 1.5l EcoBoost, FWD 6-speed manual | 150 | 195 (121) | 9.7 | 9.7 | 7.9 (35.7) | 5.4 (52.3) | 6.3 (44.8) | 145 |
| 1.5l EcoBoost, AWD 6-speed auto | 182 | 200 (124) | 10.1 | N/A | 9.4 (30.0) | 6.3 (44.8) | 7.5 (37.7) | 173 |

* In 4th gear. ØFord test figures. ØØ The declared Fuel/Energy Consumptions, CO2 emissions and electric range are measured according to the technical requirements and specifications of the European Regulations (EC) 715/2007 and (EC) 692/2008 as last amended. Fuel consumption and CO2 emissions are specified for a vehicle variant and not for a single car. The applied standard test procedure enables comparison between different vehicle types and different manufacturers. In addition to the fuel efficiency of a car, driving behaviour as well as other non-technical factors play a role in determining a car's fuel/energy consumption, CO2 emissions and electric range. CO2 is the main greenhouse gas responsible for global warming.

### SafETY AND SECURITY

| SECURITY | Standard electronic immobiliser CGEA 1.1 PATS; remote controlled central locking system including trunk lid. Remote controlled double locking system and CAT1 alarm as option. Key-Free system available as option (Keyless Start is standard). |
| --- | --- |
| SAFETY | Front air bags (driver, passenger, and driver knee), pelvis and thorax side air bags, full-length inflatable side curtains. Crash Sensing system with front crash sensor and side crash pressure sensors, pyrotechnic pretensioner and load-limiting retractor for driver and front passenger safety belts, collapsible pedals, ABS with mechanical brake assist and ESP. Titanium spec models offer Ford Stability Control features including Roll Stability Control and Curve Control. Rigid occupant protection cell and energy-absorbing crumple zones. Three-point automatic safety belts are available on all seats and 4-way adjustable headrests are standard on all front seats. ISOFIX brackets are available for the two outer backseats. |

Note: This information was correct at the time of going to print. However, Ford policy is one of continuous product development. The right is reserved to change these details at any time.

# # #

About Ford Motor Company

Ford Motor Company is a global automotive and mobility company based in Dearborn, Michigan. With about 203,000 employees and 67 plants worldwide, the company’s core business includes designing, manufacturing, marketing and servicing a full line of Ford cars, trucks and SUVs, as well as Lincoln luxury vehicles. To expand its business model, Ford is aggressively pursuing emerging opportunities with investments in electrification, autonomy and mobility. Ford provides financial services through Ford Motor Credit Company.  For more information regarding Ford and its products and services, please visit www.corporate.ford.com.

Ford of Europe is responsible for producing, selling and servicing Ford brand vehicles in 50 individual markets and employs approximately 53,000 employees at its wholly owned facilities and approximately 68,000 people when joint ventures and unconsolidated businesses are included. In addition to Ford Motor Credit Company, Ford Europe operations include Ford Customer Service Division and 24 manufacturing facilities (16 wholly owned or consolidated joint venture facilities and 8 unconsolidated joint venture facilities). The first Ford cars were shipped to Europe in 1903 – the same year Ford Motor Company was founded. European production started in 1911.

Contacts:; Jeremy Schofield. Ford of Europe. +49 221 903 0395. jschof14@ford.com.