Table of Contents

Introduction	5

Laser scanner technology	5

How does a Laser Scanner work?	5

Imager Technology	5

What is an imager?	5

How does an imager work?	6

Linear and array imager differences	6

What’s the difference between a Linear and Array Imager?	6

What imager technology is included in corded NCR Handheld scanners?	7

Handheld vs. Hands Free scanners	7

How do I know a customer requires a Handheld model vs. a Hands Free/Presentation scanner?	7

Corded vs. Cordless Handheld scanners	8

How do I know when a customer requires a corded vs. cordless handheld scanner?	8

2356-xxxx Zebra/Symbol Handheld Scanners	8

1D (Laser Scanners)	8

Which corded Handheld Scanners read 1D barcodes and include laser technology?	8

2D (Array Imagers)	8

Which corded Handheld Scanners read barcodes from mobile phones (2D)?	8

Connecting 2356-xxxx models to NCR 78xx Bioptic Scanners	9

Cables for 2356-xxxx to 78xx USB AUX port	9

What cables do I use to connect Handheld scanners to NCR Bi-optic scanners (7878/7884/7879) via USB Auxiliary port?	9

Programming instructions for 2356-xxxx to 78xx USB AUX port	9

Where are the programming instructions for 2356-xxxx models connected to NCR bioptic scanners (78xx) via USB Auxiliary Port?	9

Cables for 2356-xxxx to 78xx RS232 AUX port	10

What cables do I use to connect Handheld scanners to NCR Bi-optic scanners (7878/7884/7879) via RS232 Auxiliary port?	10

Programming instructions for 2356-xxxx HH scanner to 78xx RS232 AUX ports	10

Where are the programming instructions for 2356-xxxx models connected to NCR bioptic scanners (78xx) via RS232 Auxiliary Port?	10

POS terminal interface cables	10

Where do I find list of POS terminal interface cables.	10

EAS Tag Deactivation	11

Do any 2356 corded models support EAS tag deactivation?	11

How does a customer implement EAS tag deactivation with the 2356-9208 and 2356- 9209 models?	11

Power usage	11

What is power usage of 2356-4209 (Zebra/Symbol model DS4208)?	11

What is power usage of 2356-9208-0000 (Zebra/Symbol model DS9208)?	11

What is power usage of 7837-4010-9090 (Honeywell Xenon 1902G Cordless)	12

Drivers/Programming	12

Where do I find Driver and Programming instructions?	12

OPOS	12

How should a 2356 Handheld scanner be connected to OPOS?	12

IBM USB	12

Technical Specifications	13

Where do I find datasheets or technical specs for all 2356 corded Handheld scanner models?	13

Meantime Between Failure details	14

What is the MTBF for the 2356-9208-0000?	14

See detail in attachment.	14

What is the MTBF for the 2356-4209-0000?	14

See detail in attachment.	14

Operating Systems	14

What Operating Systems are certified with 2356 Handheld Scanners?	14

Zebra/Symbol cordless Handheld scanner	14

Do we offer the DS6878 cordless HH scanner?	14

What are battery specifications for DS6878?	15

FastLane/SSCO	16

What are the 2356-xxxx models certified to connect to 78xx bioptic scanners on FastLane/SSCO lanes?	16

Optional Hand Scanner Mount	16

What is the cordless Handheld Scanner model certified for the FastLane/SSCO Remote Attendant Stations?	16

Service part numbers	17

What are the service part numbers for the 2356 corded Handheld Scanners?	17

Warranty	17

What is the warranty for 2356 corded Handheld Scanners?	17

Honeywell 7837 Cordless Handheld Scanner	18

What is our advocated cordless Handheld Scanner offering?	18

Wireless specs	18

What is the wireless standard?	18

What is the wireless band?	18

What is wireless range?	18

Pairing process	18

What is basic pairing process for handheld scanner with its charging base?	18

Batteries	18

What is battery type?	18

How long does it take to charge the scanner from fully discharged to fully charged when placed in your cradle / base station?	18

If the base station can be operated from 5V power what impact does this have on the charge time?	19

What is the life span of the batteries?	19

What is number of scans per charge?	19

What are expected hours of operation?	19

What is process for replacing the battery?	20

Can third party batteries be used?	20

Meantime Between Failure	20

What is meantime between failure (MTBF)?	20

Power Consumption	20

How does the power saving mode work?	20

What is typical and peak power usage (W) for the unit?	20

What is power-save / sleep mode power usage (W) for the unit (if sleep mode is available)?	21

7837-4xxx connection to 78xx bioptic scanners	21

7837-4xxx connected to 78xx USB AUX port	21

What cables are required for connection to USB Auxiliary port of NCR Bioptic scanners (78xx)?	21

Where are programming instructions for connecting the 7837-4xxx models to the USB Auxiliary Port of the NCR bioptic scanners (78xx)?	21

7837-4xxx connected to 78xx RS232 AUX port	21

What cables are required for connection to the RS232 Auxiliary port of 78xx scanners?	21

7837-xxxx Programming Instructions	22

Where do I find programming sheets for 7837 connected to RS232 Auxiliary Port?	22

Drivers	22

Where do I find Driver and Programming instructions?	22

FastLane/SelfServ Checkout Remote Attendant Station	22

What cordless Handheld Scanner is certified for the FastLane/Self Serv Checkout Remote Attendant Station (RAP)?	22

Technical Specifications and Datasheets	23

Where do I find the technical specs/datasheet?	23

Connection to NCR OPOS	23

How do I connect the 7837-4xxx Handheld scanner to NCR OPOS via RS-232 SERIAL?	23

How do I connect the 7837-4xxx Handheld scanner to NCR OPOS via USB Serial Emulation or IBM 4B USB?	23

Where do I find the Honeywell serial-USB driver and barcode?	23

Operating Systems supported	24

What Operating Systems are supported?	24

Service part numbers	24

What are the service part numbers for the 7837 cordless Handheld Scanners?	24

Warranty	24

What is the warranty for 7837cordless Handheld Scanners?	24

# Introduction

## Laser scanner technology

### How does a Laser Scanner work?

- The laser line illuminates the bar code.

- A reflected analog wave signal is created.

- The analog signal is digitized.

- The decoder processes the digital data.

- The symbology algorithm is applied.

- If applicable, verification is performed via check digit.

- The data stream is sent to the host.

## Imager Technology

### What is an imager?

An imager is a device that functions in a way similar to a digital camera.  It optically captures images, printed text, handwriting, or an object, and converts it to a digital image and can be sent to an application on a PC or other computerized device.

### How does an imager work?

- A bar code is illuminated by Light Emitting Diodes (LED).

- The scanner captures an image of the entire code.

- The decoder software finds the bar code in the image to decode.

- A symbology algorithm is applied.

- If applicable, verification is performed via check digit.

- The data stream is sent to the host.

## Linear and array imager differences

### What’s the difference between a Linear and Array Imager?

Both types are essentially the same in that they are capturing a picture like a digital camera. Light Emitting Diodes, or LEDs, are used for illumination of the barcode, similar to a flash on a camera.

Once the barcode is illuminated, an array of pixels detects light from the image of the barcode.  Each pixel is an individual photo sensor that is looking at different parts of the object in front of it.  An array imager is a 2 dimensional array of pixels that can detect an image of an entire barcode, whereas a linear imager has a single row of pixels so it can capture an image of a thin slice of barcode.

### What imager technology is included in corded NCR Handheld scanners?

NCR 2D Handheld Scanners include array imagers in order to take advantage of Omni-directional scanning, capturing images and the ability to read 2D barcodes.  See table below for differences between the scanning technologies.

## Handheld vs. Hands Free scanners

### How do I know a customer requires a Handheld model vs. a Hands Free/Presentation scanner?

- It is crucial to understand the customer’s environment and how they will use the devices.

- With handheld scanners, you bring the scanner to the item.

- With hands-free scanners, you bring the items to the scanner.

- Use the 80/20 rule to decide which option is best – what is going to be done more often with the device?

- Second consideration is regarding how the scanners will be used.

- When scanning large bulky items that cannot be easily moved; a handheld scanner is recommended.

- If small items are being presented to the scanner, a hands free device is recommended.

## Corded vs. Cordless Handheld scanners

### How do I know when a customer requires a corded vs. cordless handheld scanner?

Third consideration takes another look at how the scanner will be used to determine if the scanner should be cordless.  If there is a lot of maneuvering to be done while using the scanner, a cordless scanner may be the next fit, since the user has the freedom to move closer to the barcode.  The size of the item to be scanned should also be considered when considering a corded vs. cordless solution.

If large bulky items will be scanned a cordless device is the optimal solution allowing the scanner to be brought to the item.    There may also be safety considerations that would determine if corded or cordless solutions would be more appropriate.

# 2356-xxxx Zebra/Symbol Handheld Scanners

## 1D (Laser Scanners)

### Which corded Handheld Scanners read 1D barcodes and include laser technology?

The following 2356 Hand held laser scanners read 1D barcodes.

    - 2356-1203-0000 Symbol LS1203-CR10001R with NCR logo (beige)

    - 2356-2201-0000 Symbol LS2208-SR20007R with NCR logo (charcoal)

    - 2356-2208-0000 Symbol LS2208-SR20001R with NCR logo (beige)

    - 2356-4201-0000 Symbol LS4208-SR20007ZZR with NCR logo (charcoal)

    - 2356-4209-0000 Symbol LS4208-SR20001ZZR with NCR logo (beige)

## 2D (Array Imagers)

### Which corded Handheld Scanners read barcodes from mobile phones (2D)?

The following 2356 Handheld Scanners include array imagers and can read both 1D and 2D barcodes from mobile phones.

    - 2356-4209-0000 Motorola DS4208-SR00007 with NCR logo (charcoal)

    - 2356-6701-0000 Motorola DS6707-SR20007 with NCR logo (charcoal)

    - 2356-6707-0000 Motorola DS6707-SR20001 with NCR logo (beige)

    - 2356-9208-0000 Motorola DS9208-SR20007 with NCR logo (charcoal) Hands Free

    - 2356-9209-0000 Motorola DS9208-DL20007 with NCR logo (charcoal)Hands Free

    - 2356-9808-0000 Motorola DS9808-DL00007 with NCR logo (charcoal) Hands Free

# Connecting 2356-xxxx models to NCR 78xx Bioptic Scanners

## Cables for 2356-xxxx to 78xx USB AUX port

### What cables do I use to connect Handheld scanners to NCR Bi-optic scanners (7878/7884/7879) via USB Auxiliary port?

- Follow instructions posted on ncr.com support drivers and patches via the following link:

http://www5.ncr.com/support/support_drivers_patches.asp?Class=External/Peripherals/Scanner/2356/display_Symbol

- The 2356 scanner (any model) must be programmed if is to be connected to the RealScan 74, 84, OR 78 through the Auxiliary USB port. The same cable can be used for all 2356 models.  Choose one from the chart below:

| 1432-C002-0025 | USB Cable: Series A Connector, 7ft. Straight (497-0452581) | Symbol CBA-U01-S07ZAR |
| --- | --- | --- |
| 1432-C307-0045 | USB Cable Series A connector, 15ft Straight (497-0462024) | Symbol CBA-U10-S15ZAR |
| 1432-C359-0045 | USB Powered Cable: Type A Connector, 4.5m (15ft) coiled (497-0467443) | Preferred cable for connection from 7878 bi-optic to 2356 when used in NCR SelfServ Checkout. Symbol: CBA-U09-C15ZAR |
| 1432-C392-0040 | USB Cable: Series A connector, 7ft Straight (497-0472061) *****DS4208 AND DS9208 ONLY***** | Symbol CBA-U21-S07ZAR *****DS4208 AND DS9208 ONLY***** |

## Programming instructions for 2356-xxxx to 78xx USB AUX port

### Where are the programming instructions for 2356-xxxx models connected to NCR bioptic scanners (78xx) via USB Auxiliary Port?

http://www5.ncr.com/support/support_drivers_patches.asp?Class=External/Peripherals/Scanner/2356/display_Symbol

## Cables for 2356-xxxx to 78xx RS232 AUX port

Please note that there is no 2D support for Handheld Scanners when connected the RS232 AUX port of the 78xx scanners.

### What cables do I use to connect Handheld scanners to NCR Bi-optic scanners (7878/7884/7879) via RS232 Auxiliary port?

Use cable 1432-C024-0025 (497-0461228) http://www5.ncr.com/support/support_drivers_patches.asp?Class=External/Peripherals/Scanner/2356/display_Symbol

## Programming instructions for 2356-xxxx HH scanner to 78xx RS232 AUX ports

### Where are the programming instructions for 2356-xxxx models connected to NCR bioptic scanners (78xx) via RS232 Auxiliary Port?

http://www5.ncr.com/support/support_drivers_patches.asp?Class=External/Peripherals/Scanner/2356/display_Symbol

Use following instructions when connecting a 2356-220x model to 78xx via RS232:

Use following instructions when connecting a 2356-42xx/67xx model to 78xx via RS232:

In addition, use the following programming sheet:

## POS terminal interface cables

### Where do I find list of POS terminal interface cables.

These are documented in the Ordering and Configuration Guide which can be found via the following link:

https://intranet.ncr.com/index.php?option=com_content&view=article&id=284378&assetDetail=1&Itemid=188

## EAS Tag Deactivation

### Do any 2356 corded models support EAS tag deactivation?

Yes, there are 2 corded models which incorporate an EAS tag deactivation

  - 2356-9208-0000  Motorola model DS9208 2D Presentation Imager

  - 2356-9808-0000  Motorola model DS9808 2D Presentation Imager Hybrid with DL parsing included

      - These multi-purpose imagers also incorporates an EAS tag deactivator — so a single swipe can simultaneously decode an item’s bar code and deactivate the security tag for added productivity gains.

      - The programmable decode range allows you to closely match decode and EAS deactivation ranges to help ensure that all tags are deactivated with a single scan, preventing erroneous alarms at the checkout stand that not only waste security staff time but also embarrass customers — threatening the customer experience.

### How does a customer implement EAS tag deactivation with the 2356-9208 and 2356- 9209 models?

  - Because there are several Checkpoint EAS systems available, your local Checkpoint representative should connect the digital scanner to the Checkpoint EAS system and tune the system.  To contact your local Checkpoint representative inside the U.S. call 800-257-5540, ext. 4300.  Outside the U.S., call (609)848-1800, ext. 4300.

## Power usage

### What is power usage of 2356-4209 (Zebra/Symbol model DS4208)?

5 VDC ± 10% @ 275 mA (RMS typical)

Baseline measurement on USB with default settings.  These measurements are averages measured from a 5V source at room temp:

•	Idle (triggered mode)                                    = 38mA

•	Idle (auto-aim mode, aim off) =  116mA

•	Idle (auto-aim mode, aim on) =130mA

•	Scanning (aim enabled)                                 = 270mA

•	Scanning (aim disabled)                                = 220mA

### What is power usage of 2356-9208-0000 (Zebra/Symbol model DS9208)?

Power consumption during sleep or low power state if 120ma/1.60 Watts

Baseline measurement on USB with default settings.  These measurements are averages measured from a 5V source at room temp:

Idle (IR prox on, green LEDs on)  =  140mA

Scanning                                                    = 220mA

### What is power usage of 7837-4010-9090 (Honeywell Xenon 1902G Cordless)

Typical

Full Power (100%) 
[2.5mW, 4dBm]

Medium Power (35%) [.875mW, 0dBm],

Medium Low Power (5%) [.125mW, -9dBm],

Low Power (1%) 
[.025mW, -16dBm).

## Drivers/Programming

### Where do I find Driver and Programming instructions?

Via the NCR support site (Drivers and  Patches):

http://www5.ncr.com/support/support_drivers_patches.asp?Class=External/Peripherals/Scanner/2356/display_Symbol

## OPOS

### How should a 2356 Handheld scanner be connected to OPOS?

Via IBM USB.

See Support Site for details.

http://www5.ncr.com/support/support_drivers_patches.asp?Class=External/Peripherals/Scanner/2356/display_Symbol

### IBM USB

- Unique way of handling label IDs

  - If App does not use parsing provided by OPOS

    - SSCO

    - Profile option provided – Label ID

      - Default is IBM USB

      - Set to NCR or none

  - JavaPOS can be supported with RPSW

    - Custom install setting

  - OPOS profile

    - 2x/3x profile – OPOS chooses

      - 2X does not support IBM USB

      - Choose with service property

      - 3X set for NCR OPOS Service Object NCR

## Technical Specifications

### Where do I find datasheets or technical specs for all 2356 corded Handheld scanner models?

- 2356-1203-0000 (Symbol model LS1203)

- 2356-2201-0000 & 2356-2208-0000 (Symbol model LS2208)

- 2356-4201-0000 & 2356-4208-0000 (Symbol model LS4208)

- 2356-4209-0000 (Symbol model DS4208)

- 2356-6701-0000 & 2356-6707-0000 (Symbol model DS6707)

- 2356-9208-0000 (Symbol model DS9208)

- 2356-9209-0000 (Symbol model DS9208 with DL parsing)

Same as above with Driver’s License Parsing built into unit.

- 2356-9808-0000 (Symbol model DS9808)

## Meantime Between Failure details

What is the MTBF for the 2356-9208-0000?

See detail in attachment.

## What is the MTBF for the 2356-4209-0000?

See detail in attachment.

## Operating Systems

### What Operating Systems are certified with 2356 Handheld Scanners?

Zebra/Symbol CDC Driver supports Windows 7 (32) bit and 64 bit Operating Systems

See following link for details:

http://www5.ncr.com/support/support_drivers_patches.asp?Class=External/Peripherals/Scanner/2356/display_Symbol

## Zebra/Symbol cordless Handheld scanner

### Do we offer the DS6878 cordless HH scanner?

Yes, as a customer specific TPP PID managed by the TPP team.  Model includes the Symbol logo.

If required for your customer, submit a TPP request for approval via website http://rsg.ncr.com/tpp.

  - Required PIDs for standard mode:

    - 7932-1872-0000

      - Motorola model DS6878 with Motorola/Symbol Logo

    - 2356-4272-0000

      - Charging Cradle/Base

        - STB4278-C0007WR STB 4278: CRD:42XX, CHGR,MULTIFACE/BT, TWL BLK

    - Interface cable

      - USB Shielded cables

        - 1432-C392-0040

          - CBA-U21 USB Cable, Shielded, Straight, 7'

        - 1432-C422-0028

          - CBA-U32Cable - Shielded USB: Series A Connector, 9ft. (2.8m), Coiled

        - 1432-C407-0028

          - CBA-U23 Cable - Shielded USB: Power Plus Connector, 7ft. (2.8m), Straight

        - 1432-C408-0046

          - CBA-U29 Cable - Shielded USB: 15ft. (4.6m), Coiled

      - RS232

  - Required PIDs for Presentation/Hands Free Mode:

    - 7932-1872-0000 DS6878 HH scanner

    - 7933-1006-0000 Presentation Cradle

    - 7932-1909-0000 Power Supply

    - 1449-C008-0023 AC Power Cord

    - Interface cable

      - USB

      - RS232

### What are battery specifications for DS6878?

See attached.

## FastLane/SSCO

### What are the 2356-xxxx models certified to connect to 78xx bioptic scanners on FastLane/SSCO lanes?

The USB port on the 7878/7884 scanner can be used to connect a hand-scanner to the 7350.  The following hand scanners are supported:

NCR Logo Symbol Hand Held Scanners

2356-2208-0000		NCR Logo'd - Symbol LS2208-SR20001R 0 to 17 inch range Performance Scanner, RS23, Keyboard Wedge, USB, IBM 468X/9X, White only

2356-4208-0000		NCR Logo'd - Symbol LS4208-SR20001ZZR 0 to 19 inch range High Performance Rastering Scanner, RS23, Keyboard Wedge, USB, IBM 468X/9X, White only

1432-C359-0045		Cable for Above: 2356 Scanner 7878 USB

The NCR 2356 is a wired hand-held scanner that can plug into the USB scanner port on the 7878.  The 2356 can then be used by a consumer or cashier to scan items.  When the 7878 scanner is disabled by the NCR SelfServ Checkout application, the 2356 will also be disabled.

### Optional Hand Scanner Mount

| Product ID | Functional Product Description | MRP | Plant Std Interval |
| --- | --- | --- | --- |
| AK 7346-K801 | Kit, NCR SelfServ Checkout Hand Scanner Mount | $175 | Online |

### What is the cordless Handheld Scanner model certified for the FastLane/SSCO Remote Attendant Stations?

See Honeywell section of document for 7837-xxxx model, charging cradle and interface cable information.

## Service part numbers

### What are the service part numbers for the 2356 corded Handheld Scanners?

## Warranty

### What is the warranty for 2356 corded Handheld Scanners?

All 2356 HH models include a 3 year depot warranty

# Honeywell 7837 Cordless Handheld Scanner

### What is our advocated cordless Handheld Scanner offering?

7837-4010-9090 Honeywell Xenon 1902G Extended Range 2D Wireless Scanner

7837-4011-9090 Charging/Communications Base for Xenon 1902G-ER 2D Wireless Scanner

## Wireless specs

### What is the wireless standard?

Bluetooth Class 2, v2.1

### What is the wireless band?

2.4 to 2.5 GHz (ISM Band) Adaptive Frequency Hopping Bluetooth v2

### What is wireless range?

Class 2: 10m (33’) line-of-sight

## Pairing process

### What is basic pairing process for handheld scanner with its charging base?

Turn off power before connecting a base, then power up the computer once the base is fully connected. When the base is connected and powered up, put the scanner in the base to establish a link. The green LED on the base flashes to indicate the scanner’s battery is charging.  If the scanner and base have previously been linked, you do not receive any feedback. If this is the first time that the scanner and base are linked, both devices emit a short chirpwhen their radios link. At this point, that one scanner is linked to one base.

## Batteries

### What is battery type?

1800 mAh Li-ion minimum

### How long does it take to charge the scanner from fully discharged to fully charged when placed in your cradle / base station?

Time to re-charge fully:4 to 4.5 hours

Expected operation time: 14-16 hours

### If the base station can be operated from 5V power what impact does this have on the charge time?

Up to 50,000 scans per full charge

### What is the life span of the batteries?

Battery life and capacity is subjective parameter, depending much on use case, scanner configuration, ambient environment and battery treatment.

The battery warranty is 1 year, and under normal condition a battery can be used for 2 years or more.

In an office environment batteries have exceeded 5 years of life.

### What is number of scans per charge?

Up to 57000 scans for linear and 50000 scans for 2D per full charge

### What are expected hours of operation?

Expected Hours of Operation 14-16 hours

How was battery capacity tested by Honeywell?

In order being able to indicate the battery capacity, a test was done to gather reference material.

The linked bar code set-up sheet allows to conduct a pre-defined endurance test for a fully charged battery, with bearable beeper settings. If scanned into an excel sheet, the amount of scans can be easily be counted and the battery life can be calculated from there. With the settings from the set-up sheet and fully charged battery the performance can reach up to 72.000 scans / 12 hours in a ~20°C environment.

It's a customer decision to indicate if the battery performance has declined below acceptable. Honeywell currently sets the threshold for a worn-out battery indication to 48.000 scans / 8 hours with the above pre-defined set-up.

NOTE:

On data sheets its specified a practical 14-16 hours, as a scan operator will not continuously scan, has brakes etc.

As battery capacity will decline overtime, this allows customers still working a full 8 hour shift with a slightly aged battery.

There are differences between linear and 2D scanner models, where a 2D scanner consumes roughly 15% more

The radio consumes as considerable amount of power. Scanning a bar code just prior to the scanner will go into a lower power states keeps the radio 100% of the time active, but results in a much lower amount of scans per full charge.

### What is process for replacing the battery?

.  Battery may be removed via thumbscrew on end cap in handle.

### Can third party batteries be used?

Use only NCR Li-ion battery packs, part number 100000495, rated 3.7 Vdc, 7.4Whr, in this device. Use of any non-NCR battery may result in damage and will affect warranty.

## Meantime Between Failure

### What is meantime between failure (MTBF)?

The calculated MTBF using the Bellcore part count method is 693,978 hours.

## Power Consumption

### How does the power saving mode work?

Bar code may be scanned to set the scanner’s power output to Full Power (100%) [2.5mW, 4dBm], Medium Power (35%) [.875mW, 0dBm], Medium Low Power (5%)[.125mW, -9dBm], or Low Power (1%, .025mW, -16dBm).

When there is no activity within a specified time period, the scanner enters low power mode. Scan the appropriate scanner power time-out bar code to change the time-out duration (in seconds). Scanning zero (0) is the equivalent of setting no time-out.

If there are no trigger pulls during the timer interval, the scanner goes into power down mode. Whenever the trigger is enabled, the timer is reset. If the scanner is placed in the charge base cradle and the battery is in the process of being charged, the scanner will not go into power down mode.

When the scanner is in power down mode, pull the trigger to power the unit back up. There will be a set of power up beeps and a delay of up to a few seconds for the radio to join

### What is typical and peak power usage (W) for the unit?

Full Power (100%) [2.5mW, 4dBm]

Medium Power (35%) [.875mW, 0dBm],

### What is power-save / sleep mode power usage (W) for the unit (if sleep mode is available)?

Medium Low Power (5%) [.125mW, -9dBm],

Low Power (1%) [.025mW, -16dBm).

# 7837-4xxx connection to 78xx bioptic scanners

## 7837-4xxx connected to 78xx USB AUX port

### What cables are required for connection to USB Auxiliary port of NCR Bioptic scanners (78xx)?

http://www5.ncr.com/support/support_drivers_patches.asp?Class=External/Peripherals/Scanner/7838/display_HHP

- The Xenon 1902G model (7837-4xxx) requires 1432-C413-0030 (497-0474213)

- The other 7837 and 2357 HHP models require cable 1432-C784-0025 (497-0449257)

### Where are programming instructions for connecting the 7837-4xxx models to the USB Auxiliary Port of the NCR bioptic scanners (78xx)?

http://www5.ncr.com/support/support_drivers_patches.asp?Class=External/Peripherals/Scanner/7838/display_HHP

## 7837-4xxx connected to 78xx RS232 AUX port

### What cables are required for connection to the RS232 Auxiliary port of 78xx scanners?

Please note that there is no 2D support for Handheld scanners when connected to the RS232 AUX port of the 78xx scanners.

http://www5.ncr.com/support/support_drivers_patches.asp?Class=External/Peripherals/Scanner/7838/display_HHP

- 7837-1000 OR 7837-1300, 2357 (all models), 3800g

  - Use cable 1432-C445-0025 (497-0412967)

- 7837-1200 model

  - Use cable 497-0436669

## 7837-xxxx Programming Instructions

### Where do I find programming sheets for 7837 connected to RS232 Auxiliary Port?

http://www5.ncr.com/support/support_drivers_patches.asp?Class=External/Peripherals/Scanner/7838/display_HHP

## Drivers

### Where do I find Driver and Programming instructions?

Via the NCR support site (Drivers and  Patches):

http://www5.ncr.com/support/support_drivers_patches.asp?Class=External/Peripherals/Scanner/7838/display_HHP

## FastLane/SelfServ Checkout Remote Attendant Station

### What cordless Handheld Scanner is certified for the FastLane/Self Serv Checkout Remote Attendant Station (RAP)?

A Wireless Hand Scanner can also be connected to the Attendant Station for the attendant to handle large items for the consumer.  The following hand held scanner, charging cradle and interface cables are supported:

NCR Solution:

7837-4010-9090	Xenon 1902G Extended Range 2D Wireless Scanner

7837-4011-9090	Charging/Communications Base for Xenon 1902G-ER 2D Wireless Scanner

1432-C412-0030  	CBL-130-300-S00 Cable:  RS232, black, DB9 female, with 12V Power from POS, 3m (9.8’), straight; For 12v to 5 v conversion.

1432-C414-0030	CBL-532-300-S00 STD Cable: USB, black, 12V host power with 12V to 5V conversion, locking, 3m (9.8'), straight, 5V host power

## Technical Specifications and Datasheets

### Where do I find the technical specs/datasheet?

7827-4010-9090 is based on the Honeywell Xenon 1902G.

Link to specs from Honeywell’s website:

http://www.honeywellaidc.com/en-US/Pages/Product.aspx?category=wireless-barcode-scanner&cat=HSM&pid=1902

## Connection to NCR OPOS

### How do I connect the 7837-4xxx Handheld scanner to NCR OPOS via RS-232 SERIAL?

Follow instructions in link below or attachment.

http://www5.ncr.com/support/support_drivers_patches.asp?Class=External/Peripherals/Scanner/7838/display_HHP

### How do I connect the 7837-4xxx Handheld scanner to NCR OPOS via USB Serial Emulation or IBM 4B USB?

Use OPOS Profile setup and 7837 programming barcodes via link or attachment.

http://www5.ncr.com/support/support_drivers_patches.asp?Class=External/Peripherals/Scanner/7838/display_HHP

### Where do I find the Honeywell serial-USB driver and barcode?

Use the link below:

http://www5.ncr.com/support/support_drivers_patches.asp?Class=External/Peripherals/Scanner/7838/display_HHP

## Operating Systems supported

### What Operating Systems are supported?

See link below for access to Honeywell Drivers released to support various Operating Systems.

http://www.honeywellaidc.com/en-US/Pages/Product.aspx?category=wireless-barcode-scanner&cat=HSM&pid=1902

## Service part numbers

### What are the service part numbers for the 7837 cordless Handheld Scanners?

## Warranty

### What is the warranty for 7837cordless Handheld Scanners?

All 7837 HH models include a 3 year depot warranty