[image: image1.jpg]NUVeLLl”

MOBILE LIFESTYLE EVOLUTION

Blustooth 2.1 + EDR Teohnology

WIRELESS STEREO HEADPHONES

©Biuvetooti

User's Guide




Bluetooth Accessories | NUV2882
Wireless Stereo Headphones
User’s Guide

Index (EN)-------------------------------------------------------------------------------

Index------------1

Package contents------------2

Overview------------3

Welcome------------4

Operating Instructions------------5 - 12

    -- demonstration------------5

    -- charging------------6

    -- power on/off------------6

    -- pairing------------7

    -- volume adjustment------------8

    -- playing stereo music------------9

    -- answering/ending phone calls------------9

    -- control overview------------10,11

    -- low-power prompt------------10

    -- pairing with other Bluetooth® devices------------12

Important information------------13,14

Glossary------------15

Specifications------------16

Conformity Declaration------------17
Package Contents (EN) --------------------------------------------

[image: image2.jpg]ol =

Main unit USB Extension Cable
@
DC adapter 2 pairs of earbuds

N

User's Guide





Main unit

USB Extension Cable
Charge AC/DC adapter
2 pairs of Earbuds
User's Guide
The accessories may be subject to change for the sake of product 
improvement. Changes may be made without prior notice.
Functional Parts (EN) --------------------------------------------

Front

Rear

Vol＋

Vol－ 

LED indicator

AC/DC Jack

Forward
Backward

Earphone

MFB *

Bottom 

Microphone 

* Multi-function button: Power/Pairing/Talk/Reject/Play, etc.
[image: image3.jpg]Front

Forward
Backward

Earphone

LED indicator





Welcome (EN) --------------------------------------------

Thank you for your purchase of our NUV2882 Portable Stereo Headphones.  
This User's Guide will help you to learn the basic features. Please read it carefully before using.

You will find the NUV2882 to be convenient, compact and easy to use. 

It’s both a handsfree headset and a pair of wireless stereo headphones, 

perfect for all your favorite music files. Please note that your Bluetooth® music
source must be able to transmit A2DP stereo - if not,you may need an accessory audio adapter. If you’re not familiar with some of the technical terms in this Guide, please consult the glossary on page 15.
Wearing the 

NUV2882
[image: image4.jpg]



Operating Instructions (EN) --------------------------------------------

Demonstration

Bluetooth® mobile phone

USB Bluetooth® Stereo Dongle

Bluetooth® enabled PDA

Bluetooth® Audio Adapter
[image: image5.jpg]Blustaoth"
mobile phone

s
2 (_ UsB Bluetootn®
Stereo Dongle

ﬁ

Bluetootn”

enabled PDA fdzpier

C
o v
)

3





Operating Instructions (EN) --------------------------------------------

Charging

Use the USB cable included in the package to 

connect the NUV2882 with any USB port. 

Charging will start automatically when the NUV2882 is 

connected and the red indicator light will turn on. It will be completed
when the red LED turns off and blue LED turns on. 

Note: Make sure you charge the unit for at least 

3 hours before first use.   

Power on/off

Press and hold the MFB for about 4 seconds to turn 

on, and two seconds to turn off.  When the NUV2882 

is turned on it will be in Standby mode.

Please note that the NUV2882 needs to be paired 

with another Bluetooth® source before it can transmit 

voice or music. See the next section.
about 4 seconds
[image: image6.jpg]



Operating Instructions (EN) --------------------------------------------

Pairing

"Pairing" means creating a trusted relationship between

two or more Bluetooth® devices.  The devices must be 

paired before they can communicate with each other.

Pairing identification is stored for later use, and the 

devices will automatically recognize each other the 

next time they are connected.  The NUV2882 will 

store up to 7 device IDs in memory, although it can 

only connect to one device at a time.

Note:  Make sure the NUV2882 is “off” before starting the Pairing procedure.

How to pair with cellphones 

1.  Place the NUV2882 within 30 cm of the device to be paired with.
[image: image7.jpg]Code: 0000

.

<30cm>

about 8 seconds




about 8 seconds

<30cm>

Code: 0000
Operating Instructions (EN) --------------------------------------------

2. Press and hold the MFB for about 8 seconds, 

    until the red and blue LEDs flash alternately. The 

    NUV2882 will now be in Pairing mode.

3. Initiate the Bluetooth® search function in the 

    cellphone. Please refer to the user's  manual of 

    the cellphone for more information.  

4. Select “NUV2882” in the device list on the phone 

    and follow the instructions to proceed. 

5. Follow the instruction to enter the pairing 

    code “0000”  The red LED indicator will stop 

    flashing if pairing is successful. The NUV2882’s 

    device ID will be stored in the device list of the 

    phone so that no pairing will be required for 

the next use.
Volume adjustment
At any time during a phone call, or while playing 

music, the volume can be changed with the Vol+ 

or Vol- keys. 
There will be a separate tone when at max. 

or min. volume.
Operating Instructions (EN) --------------------------------------------

[image: image8.jpg]Volume down

Volume up




Volume down

Volume up
Stereo Music

After pairing with a Bluetooth® device that supports 

the A2DP profile, such as a PC or a cellphone, the 

music output of those devices will be transmitted to 

the NUV2882 and the music play can be controlled 

from the NUV2882 (please refer 

to the attached control overview).

Answering/Ending phone calls

If  you are playing music from the cellphone, the 

music play will be paused when there is an incoming 

call. A beep tone will also be heard to notify that there 

is an incoming call. Press the multi-function button

once to answer the call and press it once again 

to end the call. When the call is ended, the music 

play will be automatically resumed.  
Operating Instructions (EN) --------------------------------------------------------
Controls overview
	Status
	MFB
	Volum＋
	Volum－

	
	Press 

briefly
	Press and hold
	Press briefly
	Press and hold
	Press briefly
	Press and hold

	Power off
	No function
	Power  on （around 4 seconds）
	No function
	No function
	No function
	No function

	
	
	Enter pairing mode（around 8 seconds）
	
	
	
	

	Standby
	No function or Last Number  Redial(double press)
	Power  off（around 4 seconds）
	Volume increase
	Voice activated dial


	Volume decrease
	Voice activated dial

	Pairing
	No function
	Power  off
	No function
	No function
	No function
	No function

	Incoming call 
	Answer call
	Reject call(around 2 seconds
	No function
	No function
	No function
	 No function

	Active call
	Cancel call
	Power off
	Volume increase
	Mute
	Volume decrease
	Mute


	Status
	Backward
	Forward

	
	Press briefly
	Press and hold
	Press briefly
	Press and hold

	Power off
	No function
	No function
	No function
	No function

	
	
	
	
	

	Standby
	No function
	No function


	No function
	No function

	Pairing
	No function
	No function
	No function
	No function

	Incoming call 
	No function
	No function
	No function
	No function

	Active call
	No function
	No function
	No function
	No function


Note: 
1.Voice transfer 


In a phone call, you can transfer voice from the car kit to your mobile phone by pressing Vol+ or Vol- in a same time you hear a beep. And then you can talk directly on your mobile phone instead of the car kit. To transfer the voice back to car kit, press Vol+ or Vol- again.
2.The voice activated dialing function only works with 

cellphones that support this function. Some cellphones may not support 
some or all of the above functions. Please refer to the user’s manual of your 
cellphone for more information.  
Low-power prompt

The red LED will flash and a beep sound will be heard 

from the unit when the power level is low. Please 

recharge the unit.

.   
Operating Instructions (EN) --------------------------------------------------------
Controls overview
The functions below only apply to devices supporting

the  A2DP / AVRCP profile. Consult the user's manual 

for more information. Here is the control overview for 

playing stereo music.
	Status
	MFB
	Volum＋
	Volum－

	
	Press 

briefly
	Press and hold
	Press briefly
	Press and hold
	Press briefly
	Press and hold

	Standby
	Play or

Last Number  Redial(double press)
	Power  off
	Volume increase
	Voice activated dial


	Volume decrease
	Voice activated dial

	Play
	Pause or 

Last Number  Redial(double press)
	Power  off
	Volume increase
	Voice activated dial


	Volume decrease
	Voice activated dial

	Pause 
	Play or 

Last Number  Redial(double press)
	Power off
	Volume increase
	Voice activated dial
	Volume decrease
	 Voice activated dial


	Status
	Backward
	Forward

	
	Press briefly
	Press and hold
	Press briefly
	Press and hold

	Standby
	Previous Song
	No function
	Next Song
	No function

	
	
	
	
	

	Play
	Previous Song
	No function


	Next Song
	No function

	Pause  
	Previous Song
	No function
	Next Song
	No function


Operating Instructions (EN) --------------------------------------------------------
Pairing with other Bluetooth® Devices
a. Pairing with a Bluetooth® enabled PDA 

1. Refer to the pairing procedure (1 ~5) shown 

    on page 7 and 8.
2. Initiate the Bluetooth?search function on the PDA. 

    Please refer to the user's manual of the PDA for 

    more information. 

3. Select “NUV2882” in the device list on the PDA and 

    follow the instructions to proceed.  The pairing 

    code is “0000”
b. Pairing with a Bluetooth® Dongle 

    (works with PCs or laptops)

1. Refer to the pairing procedure (1 ~5) shown on 

    page 7 and 8.

2. Initiate the Bluetooth® search function on the PC or 

    laptop. Please refer to the user's  manual of the 

    dongle for more information. 

3. Select “NUV2882” in the device list on the PC or 

    laptop and follow the instructions to proceed. The 

    pairing code is “0000”
c. Pairing with a Bluetooth® Audio Adapter ( 

    Alpha comm Enterprises’s STB-3131 and STB-381 are strongly 

    recommended)

1. Refer to the pairing procedure (1 ~5) shown on 

    page 7 and 8.

2. Initiate the paring function of the Bluetooth® audio 

    adapter. Please refer to the user's  manual of the 

    adapter for more information. 

3. The red LED indicator on NUV2882 will stop 

    flashing when pairing is successful.  
Important Information (EN) --------------------------------------------------------
(Note:  Please read all this information before 

using the NUV2882 Bluetooth® Stereo Headphones)


1. Check for any exceptions or rules, due to 

     national/local requirements or limitations, in usage 

     of these Bluetooth®  Stereo Headphones or any other 

     Bluetooth® devices.

2.  Please note that only authorized dealers 

     or service partners should 
remove and/or replace 

     the rechargeable battery, or any other components.


3.  Please use original accessories only. 

     Use of non-original accessories may 

     result in damage, loss of performance to the 

     headphones, injury, 
electric shock or even fire.


4.  Please do not expose the Stereo Headphones to 

     extreme high or low temperatures,  moisture 

     or fluid.


5.  Please do not drop, throw or bend the Stereo 

     Headphones as rough treatment will damage them .


6.  If the Stereo Headphones will not be used for 

     a while, please store them in a place where it is 
dry, 

     free from dust, and not prone to extreme 

     temperature fluctuations.


7.  RF frequency and energy may affect some 

     electronic systems in a motor vehicle. Please 

     check with the manufacturer to ensure that the 

     headphones will not affect the electronic systems 

     in the motor vehicle.

Important Information (EN) --------------------------------------------------------
8.  Some medical electronic equipment may not be 

     shielded from RF energy. Therefore, please 

     do not use the headphones near medical equipment 

     without permission.

9.  Please turn off the Stereo Headphones before 

     boarding any aircraft. To prevent any interference 

     with communication systems, please do not use 

     the Stereo Headphones while the plane is in the air.

10.Please turn off the Stereo Headphones in any area 

     with a potentially explosive atmosphere.


11.Please do not allow children to play with the 

     Stereo Headphones since it contains small 
parts.

12.Please do not expose the rechargeable battery 

     to extreme temperatures. If 
the battery is used at 

     low temperature , the battery capacity will be 

     reduced.
Glossary (EN)--------------------------------------------------------
A2DP:  Advanced Audio Distribution Profile

           This converts normal audio files, such as 

           MP3 files in a cellphone or a computer, into 

           streaming stereo music. Many new cellphones 

           are being made with this feature built-in.

AVRCP: Audio / Video Remote Control Profile

           This feature allows the user to control the 

           output of one device from a remote device.  

           For example, if you are listening to music on 

           your stereo headphones from your 

           cellphone, you can control the functions of the 

           cellphone by using the headphone controls for 

           volume, call transfer, etc.

Audio Adapter (Dongle):

           Some devices don’t support transmitting

           via Bluetooth®  or don’t support stereo sound.  

           An audio adapter, sometimes called a stereo 

           “dongle” will allow the device to transmit in 

           Bluetooth® stereo. Typical adapters will be used 

           for CD players, Mp3 players, computers or 

           iPods.  They will usually plug into a standard 

           USB port or 3.5mm jack, and allow for wireless 

           connections between many kinds of devices.
Specifications (EN)--------------------------------------------------------
Technical Specifications

Compliance: Bluetooth® version 2.1+EDR ,Class 2
Frequency range: 2.4-2.48 GHz

Talk time: Up to 9 Hours

Music playing time: Up to 6 hours

Standby time: Up to 180 hours
Charging time:2~3 hours

Supported Bluetooth® profiles:HFP, HSP, AVRCP , A2DP  

Dimensions(LxWxH): 44 x 38 x 22mm
Weight: 26g
Note:  The Stereo Headphones are connected to a 

mobile phone or other electronic device using 

radio frequency signals which cannot guarantee 


connection under all conditions or circumstances.  

Therefore, please never rely solely upon the 

headphones for essential or emergency 

communications.
Conformity Declaration (EN) -------------------------------------------------------
FCC: 
This device complies with part 15 

of the FCC rules. Operation is 

subject to the following two 

conditions: (1) This device

may not cause harmful interference, and (2) this 

device must accept any interference received, 

including interference that may cause undesired 

operation.
CE:

We declare under our sole 

responsibility that our product

Stereo headphones NUV2882 and in combination 

with our accessories, to which this declaration 

relates, is in conformity with the appropriate 

standards, EN 301489,EN 300328, and EN 60950, 

following the provisions of Radio Equipment and 

Telecommunication Terminal Equipment directive 

99/5/EC, with requirements covering EMC directive 

89/336/EEC, and Low Voltage directive 73/23/EEC.

ROHS:

All products are fully 

compliant with the ROHS Directive 

2002/95/EC. 

WEEE:

All  products are fully 

compliant with the WEEE Directive 

2002/96/EC.
[image: image9.jpg](EFE © &




[image: image10.jpg]Manufactured exclusively under license by
Alpha Comm Enterprises, Inc.

Iphia Comm

(_gprlses, Inc.

Copyright 2009

Made in China.

All brands and trademarks are the property of
their respective owners.




