	
	[image: image1.wmf]

	INSTRUCTIONS FOR USE
Pro-Ject Essential III Flexi-Range


Dear Music Lover,

Congratulations and thank you for choosing Pro-Ject Audio Systems record player. 
Your Essential III turntable has been handcrafted and rigorously tested by skilled technicians to give you years of enjoyable, trouble free service.
Please contact your dealer if you require additional assistance.

Important:

Your turntable was shipped partially disassembled in order to avoid damage to sensitive parts. Please check immediately to make sure that neither the packaging nor the device was damaged in transit. If you are missing parts or if the unit is damaged, please do not operate and contact your dealer.
Contents

Product illustrations








4

General setup
1. Platter and drive belt installation





5


2. Cartridge installation, stylus exchange





5
3. Vertical Tracking Force setting (VTF)





5-6
4. Anti-skating force adjustment






6
5. Azimuth setting








6


6. Turntable types and their functions 





7
a) ESSENTIAL III







7
b) ESSENTIAL III BLUETOOTH





8
c) ESSENTIAL III PHONO






9
d) ESSENTIAL III SB







10

e) ESSENTIAL III DIGITAL






11-12
f) ESSENTIAL III RECORDMASTER





12-13
Technical parameters








14
Troubleshooting, warranty, service






15
Exploded view of the packaging







16

Please do your part to help protect the environment: 

Disposal of packaging material:
Pro-Ject packaging is carefully designed to protect your component from damage in transit and you are strongly urged to keep the original packaging in order to safely ship or otherwise transport your turntable in the future.
However, the packaging materials were chosen to be environmentally friendly, so if you must discard the packaging, please recycle.
ESSENTIAL III FLEXI-RANGE

GENERAL INFORMATIONS

Controls, features and connections

1
PLATTER & FELT MAT
2 
DRIVE PULLEY                                     
3
DRIVE BELT
4 
TONEARM COUNTERWEIGHT            (4a) DOWNFORCE SCALE
5
TONEARM LIFT LEVER


6 
TONEARM REST                                  (66) REMOVABLE TRANSPORT LOCK
7 
TONEARM TUBE
8
HEADSHELL WITH FINGER LIFT AND PRE-MOUNTED ORTOFON OM10
9 
ANTI-SKATING WEIGHT WITH WIRE
10 
ANTI-SKATING WEIGHT SUPPORT HOOP
11 
ANTI-SKATING STUB WITH ADJUSTMENT SCALE
12 
LID
13 
LID HINGES
14
HINGE FASTENERS
15
POWER SUPPLY SOCKET

Accessories

1
ANTI-SKATING WEIGHT 
2 
HOOK TOOL FOR SPEED CHANGE                                     
3
SINGLE ADAPTOR 
4
ALLEN KEY 1,5mm
5 
COUNTERWEIGHT no. 00
6
TWO-POINT CARTRIDGE ALIGNMENT PROTRACTOR
7
UNIVERSAL POWER SUPPLY
8
CONNECTION CABLE (CONNECT IT E RCA)                                  
9
INSTRUCTION FOR USE
[image: image2.jpg]



[image: image3.jpg]



SETUP

1. Platter and drive belt installation

Remove brown paper insert which holds the platter in transport position. Put the felt mat on the top of the platter. Install the drive belt as shown in the following illustration.
[image: image4.jpg]DRIVE BELT

PLATTER & FELT MAT

PAPER INSERT




2. Cartridge installation, stylus exchange
* If you purchased this turntable in a package that included the cartridge, this has already been done for you.
Install the cartridge into the aluminium headshell, using the appropriate hardware included with your cartridge. All cartridges with half inch mounting holes can be fitted. 
Connect the cartridge as indicated below:
· White

left channel L+

· Red

right channel R+

· Green

right channel R-

· Blue

left channel L-

For correct cartridge alignment, use the two-point cartridge alignment protractor provided with your Essential III accessories. If unfamiliar with two-point setup, please refer to your dealer.
A damaged or worn stylus may harm your record collection and should be replaced. 

Please remove stylus protection before pulling the stylus from the cartridge and replace for a new one.

Stylus type is exchange for pre-mounted cartridge Ortofon OM 10: Stylus OM 10. If in doubt consult your local dealer or distributor!
3. Vertical Tracking Force setting (VTF)

Before setting the Vertical Tracking Force, confirm the exact weight of your cartridge. The counterweight supplied (no. 00) is suitable for cartridges from 3,5 to 5,5g. An alternative counterweight (no. 01) for cartridges between 6 – 9g is available on request.
Pushing carefully, turn the counterweight onto the rear end of the tonearm tube. With the tonearm lift in lower position, carefully rotate the counterweight until the tonearm tube balances out. Once the tonearm is correctly balanced, return it to the rest. Hold the counterweight without moving it, and gently revolve the downforce scale ring until the zero is in line with the anti-skating stub. 

Rotate the counterweight counter clockwise (seen from the front) to adjust the downforce according to the cartridge manufacturer's recommendations. One mark on the scale represents 1 mN (= 0,1g / 0,1 Pond) of downforce.

Please note: Adjust the VTF prior to installing the anti-skating weight.

[image: image5.jpg]VERTICAL TRACKING FORCE SETTING RECOMMNDED

VTF 17mN

+VTF
-VTF

TONEARM LIFT LEVER DOWNFORCE SCALE





4. Anti-skating  force adjustment

Hang the loop of the thread of the anti-skating weight in the groove of the anti-skating stub corresponding to the downforce applied to your cartridge and feed the thread through the loop of the wire support.
The anti-skating force must be adjusted corresponding to the downforce as follows:
	VTF

            Groove in the stub
10 - 14mN

1st  from bearing rings

15 - 19mN

2nd    "         "         "

20mN and bigger
3rd     "         "         "
	[image: image6.jpg]GROOVE 1
GROOVE 2
GROOVE 3







5. Azimuth setting

The cartridge needle must be perpendicular to the record in order to trace the groove wall modulations correctly. 
The azimuth (angle) is precisely set by the factory. In the event that you need to modify this setting, however, follow the instructions below.
Loosen the small azimuth screw, again using the 1.5 mm allen key. 
[image: image7.wmf] DO NOT REMOVE THE AZIMUTH SET SCREW COMPLETELY!
Loosen the screw just enough to be able to gently rotate the arm tube and set the azimuth to the correct position. The correct position can be checked from the front view, preferably with the needle placed on a mirror placed on the platter. Once the azimuth setting is correct, gently re-tighten the azimuth setting screw.

[image: image8.jpg]AZIMUTH SETTING SCREW





ESSENTIAL III



    SPECIFIC INFORMATIONS
Controls, features and connections

1
ON/OFF SWITCH
2 
STEPPED DRIVE PULLEY                                     
3
OUTPUT CABLE CONNECT IT E
_____________________________________________________________________________________________________________

[image: image9.jpg]



Switching on and off

Pressing the ON/OFF switch alternately starts or stops the motor.
Changing replay speed

To play 45 r.p.m., put the drive belt over the larger diameter part of the motor pulley. To revert to 33 r.p.m. repeat the proceedings using the smaller step on the pulley.

Connection to the amplifier

The record player has a captive tonearm signal lead for connection to the amplifier. Use the Phono input on your amplifier. Make sure that the phono input offers correct matching and amplification for the type of cartridge used. Line inputs (such as CD, Tuner, Tape or Video) are not suitable.
Take care to connect the left and right channels correctly. The right channel is usually marked red, the left channel black or white. Check the manual supplied with your amplifier for relevant information. The earthing wire of the tonearm lead should be connected to the earth terminal on your amplifier (if provided).

If your amplifier does not have an input suitable for phono cartridges you will require a separate phono amplifier stage for MM or MC cartridges which is then connected between the record player and a free line level input of the amplifier.

ESSENTIAL III Bluetooth   
    SPECIFIC INFORMATIONS

Controls, features and connections

1
ON/OFF SWITCH
2 
STEPPED DRIVE PULLEY                                     
3
PHONO/LINE OUTPUT, EARTH CONNECTION
4 
BLUETOOTH SWITCH

_____________________________________________________________________________________________________________

[image: image10.jpg]



Switching on and off

Pressing the ON/OFF switch alternately starts or stops the motor.
Changing replay speed

To play 45 r.p.m., put the drive belt over the larger diameter part of the motor pulley. To revert to 33 r.p.m. repeat the proceedings using the smaller step on the pulley.
Connection to the amplifier

The record player has two possible outputs – Line OUT with integrated preamplifier and Phono OUT for connection to the preamplifier. If you choose Phono OUT, than use the Phono input (sometimes labelled gram, disc or RIAA) on your amplifier. Make sure that the phono input offers correct matching and amplification for the type of cartridge used. If you choose Line OUT, than use the Line inputs (like CD, Tuner, Tape or Video).
Take care about connecting the left and right channel correctly. The right channel is usually marked red, the left channel black or white. Check the manual of your amplifier for relevant information. An earthing wire may be connected to the screw terminal between the sockets if you encounter humming problems.

Bluetooth pairing 

The record player has Bluetooth output and transmit signal automatically when is connected to mains.

It is paired automatically with any device with Bluetooth receiver (such as Pro-Ject MaiA, MaiA DS, Bluetooth Box s and Bluetooth Box E). 

For pairing with other devices and portable speakers use the manual of the given product.

For restart of the Bluetooth transmitter, please disconnect the record player from mains. Restart is needed only when there is a change of the Bluetooth receiver.

ESSENTIAL III Phono                SPECIFIC INFORMATIONS

Controls, features and connections

1
ON/OFF SWITCH
2 
STEPPED DRIVE PULLEY                                     
3
PHONO/LINE OUTPUT, EARTH CONNECTION

4 
PHONO/LINE OUTPUT SWITCH
_____________________________________________________________________________________________________________

[image: image11.jpg]



Switching on and off

Pressing the ON/OFF switch alternately starts or stops the motor.
Changing replay speed

To play 45 r.p.m., put the drive belt over the larger diameter part of the motor pulley. To revert to 33 r.p.m. repeat the proceedings using the smaller step on the pulley.

Connection to the amplifier

The record player has gold plated female RCA phono sockets and possibility to choose between Phono and LINE output. With LINE OUT on, you can use a line input (such as AUX, CD, Tuner, Tape or Video) on your amplifier. With PHONO OUT on, you can use your own phono pre-amplifier (sometimes labelled gram, disc or RIAA).

[image: image12.jpg]



Take care to connect the left and right channels correctly. The right channel is usually marked red, the left channel black or white. The earthing wire of the tonearm lead should be connected to the earth terminal on your amplifier (if provided).

ESSENTIAL III SB                      SPECIFIC INFORMATIONS

Controls, features and connections

1
SPEED CONTROL
2 
DRIVE PULLEY                                     
3
PHONO/LINE OUTPUT, EARTH CONNECTION
___________________________________________________________________________________________________________[image: image13.jpg]=SSEWNT\AL





Switching on and off, changing replay speed

To play records at 33 r.p.m. press the speed control switch to starts the motor. Pressing the ON/OFF switch two times selects 45 r.p.m.. The selected speed is shown respectively by the flashing LED.  When the platter has attained full speed, the LEDs stop flashing.

A long (more than three seconds) press on the ON/OFF switch stops the motor.
Connection to the amplifier

Connect the tonearm cable provided with the accessories to the RCA tonearm output that is located at the rear  of the turntable, behind the tonearm. Connect the grounding cable on the earth connection screw.

If your amplifier does not have an input suitable for phono cartridges you will require a separate phono amplifier stage for MM or MC cartridges which is then connected between the record player and a free line level input of the amplifier.
[image: image14.jpg]LEFT CHANNEL RIGHT CHANNEL
EARTH CONNECTION




ESSENTIAL III Digital                SPECIFIC INFORMATIONS

Controls, features and connections

1
ON/OFF SWITCH
2 
STEPPED DRIVE PULLEY                                     
3
PHONO/LINE OUTPUT, EARTH CONNECTION

4 
PHONO/LINE OUTPUT SWITCH

5
DIGITAL OUTPUT SWITCH (24bit/48kHz or 24bit/96kHz)
___________________________________________________________________________________________________________
[image: image15.jpg]



Switching on and off

Pressing the ON/OFF switch alternately starts or stops the motor.
Changing replay speed

To play 45 r.p.m., put the drive belt over the larger diameter part of the motor pulley. To revert to 33 r.p.m. repeat the proceedings using the smaller step on the pulley.

Connection to the amplifier

The record player has two possibilities of connection to the amplifier. 

The analog output: 

The record player has gold plated female RCA phono sockets and possibility to choose between Phono and LINE output. With LINE OUT on, you can use a line input (such as AUX, CD, Tuner, Tape or Video) on your amplifier. With PHONO OUT on, you can use your own phono pre-amplifier (sometimes labelled gram, disc or RIAA).

[image: image16.jpg]



Take care to connect the left and right channels correctly. The right channel is usually marked red, the left channel black or white. The earthing wire of the tonearm lead should be connected to the earth terminal on your amplifier (if provided).

The digital output:
The record player has digital S/PDIF output through optical connector with switch between 24bit/48kHz and 24bit/96kHz. Use the optical input on your amplifier, AV receiver or D/A converter. Optical cable is not supplied with the record player. Your dealer can help you with selection of the optical cable.  

ESSENTIAL III Recordmaster     SPECIFIC INFORMATIONS

Controls, features and connections

1
SPEED CONTROL
2 
DRIVE PULLEY                                     
3
PHONO/LINE OUTPUT, EARTH CONNECTION

4 
PHONO/LINE OUTPUT SWITCH

___________________________________________________________________________________________________________

[image: image17.jpg]



Switching on and off, changing replay speed

To play records at 33 r.p.m. press the speed control switch to starts the motor. Pressing the ON/OFF switch two times selects 45 r.p.m.. The selected speed is shown respectively by the flashing LED.  When the platter has attained full speed, the LEDs stop flashing.

A long (more than three seconds) press on the ON/OFF switch stops the motor.
Connection to the amplifier

The record player has gold plated female RCA phono sockets (14) and possibility to choose between Phono and LINE output. With LINE OUT on, you can use a line input (such as AUX, CD, Tuner, Tape or Video) on your amplifier. With PHONO OUT on, you can use your own phono pre-amplifier (sometimes labelled gram, disc or RIAA).

[image: image18.jpg]



Take care to connect the left and right channels correctly. The right channel is usually marked red, the left channel black or white. The earthing wire of the tonearm lead should be connected to the earth terminal on your amplifier (if provided).

Connecting to a Computer

Connect the USB-output of the unit to a free USB-socket on your computer and turn it on/make sure it is powered on.
The “new hardware found assistant“ will automatically detect the unit and announces it as a Microphone USB audio codec. There is no need to install a driver manually.

After installation, one setting has to be done. For example - Windows 7® operating system:

( Control Panel ( Sound ( Recording ( Microphone USB Audio CODEC ( Properties (
Advanced: set to 2 channel, 16 bit, 48000 Hz (DVD Quality)
	[image: image19.wmf]
	Please note: Connection should be made to an USB-socket of your computer directly. Connecting to USB-hubs or switches can cause problems.


Recording Programme

In case you do not have a recording programme installed on your computer, you may download one free from the Internet. Recording programmes may be found by searching Google.
	[image: image20.wmf]
	In the menu of the recording programme, source or input may have to be set to Microphone USB audio codec.
Please be aware that downloads from the Internet are made entirely at your own risk. Under no circumstances can we carry responsibility or provide support for third-party software products sourced from the internet or for any damage or problems arising from the use thereof. All downloads should be checked with an up-to-date anti-virus programme. License fees may be applicable.


Technical specifications Pro-Ject Essential III Flexi-Range / Pro-Ject 8.6

	Nominal speeds
	33/45 r.p.m.

	Speed variance
	33: ±0,70% 45: ±0,60% 

	Wow and flutter
	33: ±0,21% 45: ±0,19%

	Signal to noise
	-66dB

	Effective tonearm mass
	8,0g

	Effective tonearm length
	8,6" (218,5mm)

	Overhang
	22,0mm

	Power consumption
	4,5W

	Outboard power supply
	15V/500mA DC, suitable for your country's mains supply

	Dimensions (W x H x D)
	420 x 112 x 330mm

	Weight
	5,0kg

	Dimensions packaging (W x H x D)
	488 x 235 x 393mm

	Weight incl. packaging
	7,0kg


Technical specifications MM-Cartridge Ortofon OM 10
	Frequency range
	20-22.000Hz

	Channel separation
	22dB/1kHz

	Output voltage
	4mV

	Recommended load impedance
	47kohms/amplifier connection – MM-input

	Compliance/stylus type
	20µm/mN – elliptical

	Recommended tracking force
	15mN

	Weight
	5g


Technical specifications MM phono pre-amplifier (Essential III Digital)
	Gain

	34dB

	Output voltage typically
	160mV/1kHz at 4mV/1kHz input

	Input impedance
	47kohms/100pF

	Noise floor
	>73dB

	RIAA-equalisation curve accuracy
	20Hz-20kHz / max. 0,5dB

	Optical output:
	24/48 kHz or 24/96 kHz (bit depth/sampling frequency)


Technical specifications MM phono pre-amplifier (Essential III Phono, RecordMaster)
	Gain

	34dB

	Output voltage typically
	200mV/1kHz at 5mV/1kHz input

	Input impedance
	47kohms/100pF

	Noise floor
	 -68dB

	RIAA-equalisation curve accuracy
	20Hz-20kHz / max. 0,5dB


Potential incorrect use and fault conditions
Pro-Ject turntables are manufactured to the highest standards and undergo strict quality controls before leaving the factory. Faults that may possibly occur are not necessarily due to material or production faults but can sometimes be caused by incorrect use or unfortunate circumstances. Therefore the following list of common fault symptoms is included.

The platter doesn't turn although the unit is switched on:

The unit is not connected to the mains power supply.

No mains at the socket.

Drive belt is not fitted or has slipped off.

No signal through one or other channel or both channels:
No signal contact from the cartridge to the internal tonearm wiring or from that to the arm lead or

from that to the phono box or between that and the amplifier. This could be due to a faulty plug,

broken wire or solder joint or simply loose plug/socket connection.

Phono input not selected at amplifier.

Amplifier is not switched on.

Amplifier or speakers are defective or muted.

No connection to the loudspeakers.

Strong hum on phono input:

No earth connection from cartridge or arm or arm cable to amplifier, or earth loop.

Distorted or inconsistent sound from one or both channels:

Record player is connected to wrong input of amplifier, or MM/MC switch incorrectly set.
Needle or cantilever damaged.

Wrong r.p.m., drive belt overstretched or dirty, platter bearing without oil, dirty or damaged
Service

Should you encounter a problem which you are not able to alleviate or identify despite the above information, please contact your dealer for further advice. Only when the problem cannot be resolved there should the unit be sent to the responsible distributor in your country.

Guarantee repairs will only be affected if the unit is returned correctly packaged. For this reason we recommend keeping the original packaging.

Never return a record player without making sure that is it safely disassembled and correctly packaged in the original packaging according to the diagrams on the last page of this user guide. Please remove these parts and pack them separately: lid, counterweight, anti-skating weight and belt.
Fit the cartridge protection cap and the transport lock for the tonearm tube, prior to carefully packaging the record player.

Warranty

	[image: image21.wmf]

	The manufacturer accepts no responsibility for damage caused by not adhering to these instructions for use and/or by transportation without the original packaging. Modification or change to any part of the product by unauthorized persons, release the manufacturer from any liability over and above the lawful rights of the customer.


	Pro-Ject Audio Systems is a Registered Trademark of                H. Lichtenegger.

This guide was produced by: Pro-Ject Audio Systems
Copyright © 2017. All rights reserved.
	The information was correct at the time of going to press. The manufacturer reserves the right to make changes to the technical specification without prior notice as deemed necessary to uphold the ongoing process of technical development. 


[image: image22.jpg]ACCESSORIES
INSTRUCTION FOR USE

D

TURNTABLE
TONEARM TRANSPORT LOCK

CCOUNTERWEIGHT

POWER SUPPLY



[image: image23.wmf]
16

15


