Smeg Oven Instructions

Left Hand Side of Oven:

Decrease time

Increase Time

End of cooking time

Cooking time

Timer

Right Hand Side of oven:

Function knob

Thermostat light

Thermostat (Temperature) knob 50oC - 250 oC

Wide Grill Element

Upper and lower heating elements

Functions: 9 options

Grill Element and ventilation

Small Grill Element

Circular Heating element and ventilation

Circular heating element + ventilation + lower heating element

Lower Heating element and ventilation

Upper and Lower heating elements and ventilation

No Selected Function

- To turn oven on:

  - Use Function knob to select desired settings (grill, heating elements, fan etc.)

  - Light inside oven should turn on

  - Select desired temperature by turning thermostat knob

  - Thermostat light should turn on

  - When selected oven temperature is reached, thermostat light will turn off

  - If oven won’t turn on, then press the cooking buttons together

Smeg Timer instructions

- To set (24 hour) clock if required

  - Press cooking buttons together

  - Immediately press +/- until correct time shows (i.e. for 2am press ‘+’ 120 times)

  - Note: Pressing both cooking buttons resets timer countdown to 0:00

- To set cooking timer countdown (oven will turn itself off after countdown)

  - Press

  - The symbol ‘A’ will flash and 0:00 will be displayed

  - Press +/- until desired cooking time is displayed (for 1 hour press ‘+’ 60 times etc.)

  - The clock will automatically reappear after 5 seconds

  - The symbol ‘A’ will show on the clock

  - Beeper will sound and oven will turn off at the end of set cooking time

  - To stop beeper, press

- To check remaining time during countdown

  - Press

  - Remaining cooking time will be displayed for 5 seconds

- To check what time cooking will finish during countdown

  - Press

  - Clock will display cooking finish time for 5 seconds

- To reset cooking timer

  - Instruction (4) and set to 0:00, or

  - Press

  - The clock will be displayed (ready for changing) and the symbol ‘A’ will disappear

  - Timer will display 0:00 next time	is pressed

- To set timer only (i.e.  beeper will sound, but oven won’t turn off)

  - Press bell button

  - 0:00 will be displayed

  - Press+/- until required time is displayed

  - Clock will automatically reappear after 5 seconds and bell symbol will be displayed

  - At end of time, beeper will sound but oven will stay on

  - Press bell button to turn off beeper

Smeg additional notes

- If defrost is required, then select ‘0’ on the bottom knob and have a fan selected with the function knob

- Do not place Al foil or tray etc on the bottom of oven

- Keep oven door closed when cooking

- When opening a hot oven, first open door 5cm for 4-5 seconds before fully opening

- After cooking, use heat protective gloves when removing hot items

- To clean oven

  - Use a damp cloth and non abrasive detergent. Rinse and dry.

  - Clean glass with absorbent kitchen paper or a damp sponge and regular detergent

  - Do not use metallic sponges or sharp scrapers