# SRTP300 Instruction Manual

Wireless tire pressure and temperature monitoring system

What is in the box

In the box you will find the following:

Waterproof main unit with multi-color display

Set of tire sensors

Set of luck-nuts

Luck-nut tool

Handlebar mount

USB charging cord

Battery changing tool

Instruction manual

# Installation of external sensor

- Screw on the luck-nut to the metal tire stem.

- Place the sensor on the stem and tighten it.

- Tighten the lock-nut counter clockwise, to hold the sensor

- Using the provided luck-nut tool, tighten the luck-nut counter clockwise

Turning the unit on or off:

Press and hold “＜” for 3s to 6s to turn the monitor on or off.

Display icons:

# Function

Default settings

High pressure 44 PSI (3.0 Bar)

Low pressure 29 PSI (2.0 BAR) 65℃

High temperature 149℉ (65℃)

High Pressure Alert:

When the pressure of any tire is higher than the pre-defined value (44 PSI), the high pressure icon will flash with an audible alarm.

Low Pressure Alert:

When the pressure of any tire is lower than the pre-defined value (29 PSI), the low pressure icon will flash with an audible alarm.

High Temperature Alert:

When the temperature of any tire is higher than the pre-defined value (149℉), the high temperature icon will flash with an audible alarm.

Fast Leakage Alert:

When front sensor detects tire leak, low pressure value and leak icon will flash with an audible alarm.

Sensor Low Battery Alert:

When the sensor battery is low, the low battery icon of corresponding tire will flash with an audible alarm.

# Parameter setting

- Maximum pressure alarm value can be set as high as 87PSI (6 BAR).

- Maximum pressure range can be displayed 99 PSI (up to 8 BAR).

High Pressure Setting:

- Press and hold SET button to enter settings.

- Press ＜ to  choose "high pressure HI" as  below,

- Press SET button to setup high pressure alarm value.

- Press＜ to adjust the value desired

- Press SET to save.

- Press and hold SET button about 3s to save and exit to standby.

Low Pressure Setting:

- Press and hold SET button to setup

- Press ＜ to  choose "low pressure LO" as below,

- Press SET button to setup low pressure alarm value.

- Press ＜ to adjust the value desired

- Press SET to save.

- Press and hold SET button about 3s to save and exit..

High Temperature Setting:

- Press and hold SET button to setup page

- Press ＜ to  choose "high temperature HI" as below

- Press SET button to setup high temperature alarm.

- Press ＜ to adjust the value desired

- Press SET to exit. Finally press and hold SET button about 3s to save and exit to standby.

Pairing Sensors:

- Press SET button five times continuously to enter pairing mode, as bellow

- Press ＜ to choose the tire you wish to pair. Pairing tire will start to flash

- For External sensors: Tighten the sensor  For Internal sensors : Inflate the tire

- The monitor will look for the sensor and displays the pressure and temperature.

- press and hold SET about 3s to save and exit to standby.

Pressure Unit Setting:

- Press and hold SET button to enter set up.

- Press ＜ to choose "BAR PSI" as below

- Press SET to setup pressure unit.

- Press ＜ to choose the pressure unit you desire

- Press SET to saves.

- Press and hold SET button about 3s to save and exit.

Temperature Unit Setting:

- Press and hold SET button to enter set up.

- Press ＜ to choose "℃℉"as below,

- Press SET to setup temperature unit.

- Press ＜ to choose the temperature unit you wish.

- Press SET to exit setup .

- Press and hold SET button for 3s to save and exit.

# Replace battery of external sensor

Caution：Sensor battery has been installed before delivery. Please do not disassemble unless the battery has to be replaced.

- Remove the safety gasket.

2. Place the sensor into the battery changing tool.

- Use battery changing tool to unscrew the sensor case.

- Remove the battery and replace with a fresh battery.

- Use the battery changing tool to close the sensor

- Replace the safety gasket.