MSI PS341WU - How to get the 5k@60Hz by Apple MacBook Pro.
Step 1 : 

Check macOS is working at the latest version, now is “macOS Catalina 10.15.1”.

Step 2: 
Connect “USB Type C to USB Type C cable” or “USB Type C to DisplayPort cable” between MacBook Pro and PS341WU. 
Step 3 : 
Press and hold the “Option” key at the MacBook Pro keyboard (Fig.1), and then click “Scaled” item In the System Preferences \ Displays of PS341WU (Fig.2) , the resolution 5120x2160 will appear, and then select the refresh rate to 30Hz, 50Hz or 60Hz.
For more timing detail supported by MacBook Pro , please refer the List 1.
Fig.1 : MacBook Pro keyboard layout.

[image: image1.png]I N O I 0 e | o B | R R B @) (0]

~dlrilef #[1 s Il % ]~ & * v -0+
‘‘‘‘‘‘‘‘‘‘‘‘

ﬁ
- [°I[WHE][RJ[T]I*HUH'H°J[PH‘ﬂﬂ’ﬂﬂ'l
(= I EE]EHEM MM MEEEDCS
- CJEMEMMEEE L

ISP EE T,





Fig.2 :  5120x2160 resolution select method.
[image: image2.png]MSI PS341WU

["Display " Color

Resolution: () Default for display

g @® scaled

: 4608 x 2592 (low resolution)
4096 x 2304 (low resolution)

3840 x 2160
3440 x 1440 (low resolution)

Rotation: [|Standard @]

Refresh Rate: |30 Hertz

Press and Hold the Option Key
then click the Scaled item.

Gather Windows ?




List 1 : MacBook Pro timing supported list.

	
	MacBook Pro 13” 
Intel Iris Graphics
	MacBook Pro 15” 2017
AMD Radeon Pro 555
	MacBook Pro 15” 2018
AMD Radeon Pro 560x

	USB C to USB C
	4K@60Hz*
	5k@50Hz**
	5K@60Hz

	USB C to Display Port
	4K@60Hz*
	5k@50Hz**
	5K@60Hz


* MacBook Pro 13” IGD video output follows DP1.2 protocol, so, the limitation is 4K@60Hz.

** MacBook Pro 15” supports 5k@60Hz starting with 2018 edition.
