www.lg.com

OWNER'S MANUAL

AIR

CONDITIONER

Read this owner's manual thoroughly before operating the appliance and keep it

handy for reference at all times.

TYPE : WALL MOUNTED

EN

English

MFL69781201

Rev.01_041817

-

LG

Life'sG ood

111111111111111

II IIII

I

II

Il

llllllll

www.lg.com

OWNER'S MANUAL

AIR

CONDITIONER

Read this owner's manual thoroughly before operating the appliance and keep it

handy for reference at all times.

TYPE : WALL MOUNTED

EN

English

MFL69781201

Rev.01_041817

-

LG

Life'sG ood

111111111111111

II IIII

I

II

Il

llllllll

TABLE OF CONTENTS

This manual may contain images or content

different from the model you purchased.

This manual is subject to revision by the

manufacturer.

SAFETY INSTRUCTIONS

...................................................

3

IMPORTANT SAFETY INSTRUCTIONS

................................................

3

OPERATION

.......................................................................

11

Notes for Operation

..............................................................................

11

Parts and Functions

..............................................................................

11

Wireless Remote Control

......................................................................

12

Restarting the Air Conditioner Automatically

........................................

17

Using the Mode Function

......................................................................

17

Using the Jet Mode Function

................................................................

19

Using the Fan Speed Function

.............................................................

20

Using the Air Flow Direction Function

...................................................

20

Setting the On/Off Timer

.......................................................................

21

Using the Sleep Function (Optional)

.....................................................

22

Using the Simple Timer Function (Optional)

.........................................

22

Using the Energy Display Function (Optional)

......................................

23

Using the Light Off Function (Optional)

................................................

23

Using the Comfort Air Function (Optional)

............................................

23

Using the Energy Saving Function (Optional)

......................................

24

Using the Energy Control Function (Optional)

......................................

24

Using Special Functions

.......................................................................

25

SMART FUNCTIONS

.........................................................

28

Using Smart ThinQ Application

.............................................................

28

MAINTENANCE

..................................................................

30

Clean the Air Filter

................................................................................

32

Clean the 3M and Triple Filter (Optional)

..............................................

33

TROUBLESHOOTING

.......................................................

34

I

I

IL_

__

TABLE OF CONTENTS

This manual may contain images or content

different from the model you purchased.

This manual is subject to revision by the

manufacturer.

SAFETY INSTRUCTIONS

...................................................

3

IMPORTANT SAFETY INSTRUCTIONS

................................................

3

OPERATION

.......................................................................

11

Notes for Operation

..............................................................................

11

Parts and Functions

..............................................................................

11

Wireless Remote Control

......................................................................

12

Restarting the Air Conditioner Automatically

........................................

17

Using the Mode Function

......................................................................

17

Using the Jet Mode Function

................................................................

19

Using the Fan Speed Function

.............................................................

20

Using the Air Flow Direction Function

...................................................

20

Setting the On/Off Timer

.......................................................................

21

Using the Sleep Function (Optional)

.....................................................

22

Using the Simple Timer Function (Optional)

.........................................

22

Using the Energy Display Function (Optional)

......................................

23

Using the Light Off Function (Optional)

................................................

23

Using the Comfort Air Function (Optional)

............................................

23

Using the Energy Saving Function (Optional)

......................................

24

Using the Energy Control Function (Optional)

......................................

24

Using Special Functions

.......................................................................

25

SMART FUNCTIONS

.........................................................

28

Using Smart ThinQ Application

.............................................................

28

MAINTENANCE

..................................................................

30

Clean the Air Filter

................................................................................

32

Clean the 3M and Triple Filter (Optional)

..............................................

33

TROUBLESHOOTING

.......................................................

34

I

I

IL_

__

SAFETY INSTRUCTIONS

EN

The following safety guidelines are intended to prevent

unforeseen risks or damage from unsafe or incorrect operation

of the appliance.

The guidelines are separated into ‘WARNING’ and ‘CAUTION’

as described below.

This symbol is displayed to indicate matters and

operations that can cause risk. Read the part with this

symbol carefully and follow the instructions in order to

avoid risk.

WARNING

This indicates that the failure to follow the instructions can

cause serious injury or death.

CAUTION

This indicates that the failure to follow the instructions can

cause the minor injury or damage to the product.

IMPORTANT SAFETY INSTRUCTIONS

WARNING

To reduce the risk of explosion, fire, death, electric shock,

injury or scalding to persons when using this product, follow

basic precautions, including the following:

Children in the Household

This appliance is not intended for use by persons (including children)

with reduced physical, sensory or mental capabilities, or lack of

experience and knowledge, unless they have been given supervision

or instruction concerning use of the appliance by a person

responsible for their safety. Children should be supervised to ensure

that they do not play with the appliance.

A

A

A

A

SAFETY INSTRUCTIONS

EN

The following safety guidelines are intended to prevent

unforeseen risks or damage from unsafe or incorrect operation

of the appliance.

The guidelines are separated into ‘WARNING’ and ‘CAUTION’

as described below.

This symbol is displayed to indicate matters and

operations that can cause risk. Read the part with this

symbol carefully and follow the instructions in order to

avoid risk.

WARNING

This indicates that the failure to follow the instructions can

cause serious injury or death.

CAUTION

This indicates that the failure to follow the instructions can

cause the minor injury or damage to the product.

IMPORTANT SAFETY INSTRUCTIONS

WARNING

To reduce the risk of explosion, fire, death, electric shock,

injury or scalding to persons when using this product, follow

basic precautions, including the following:

Children in the Household

This appliance is not intended for use by persons (including children)

with reduced physical, sensory or mental capabilities, or lack of

experience and knowledge, unless they have been given supervision

or instruction concerning use of the appliance by a person

responsible for their safety. Children should be supervised to ensure

that they do not play with the appliance.

A

A

A

A

EN

Installation

•

Do not install the air conditioner on an unstable surface or in a

place where there is danger of it falling.

•

Contact an authorized service center when installing or relocating

the air conditioner

.

•

Install the panel and the cover of the control box safely.

•

Do not install the air conditioner in a place where flammable liquids

or gases such as gasoline, propane, paint thinner, etc., are stored.

•

Make sure that the pipe and the power cable connecting the indoor

and outdoor units are not pulled too tight when installing the air

conditioner.

•

Use standard circuit breaker and fuse that conform to the rating of

the air conditioner.

•

Do not input air or gas into the system except with the specific

refrigerant.

•

Use non-flammable gas (nitrogen) to check for leak and to purge

air; using compressed air or flammable gas may cause fire or

explosion.

•

The indoor/outdoor wiring connections must be secured tightly,

and the cable should be routed properly so that there is no force

pulling the cable from the connection terminals. Improper or loose

connections can cause heat generation or fire.

EN

Installation

•

Do not install the air conditioner on an unstable surface or in a

place where there is danger of it falling.

•

Contact an authorized service center when installing or relocating

the air conditioner

.

•

Install the panel and the cover of the control box safely.

•

Do not install the air conditioner in a place where flammable liquids

or gases such as gasoline, propane, paint thinner, etc., are stored.

•

Make sure that the pipe and the power cable connecting the indoor

and outdoor units are not pulled too tight when installing the air

conditioner.

•

Use standard circuit breaker and fuse that conform to the rating of

the air conditioner.

•

Do not input air or gas into the system except with the specific

refrigerant.

•

Use non-flammable gas (nitrogen) to check for leak and to purge

air; using compressed air or flammable gas may cause fire or

explosion.

•

The indoor/outdoor wiring connections must be secured tightly,

and the cable should be routed properly so that there is no force

pulling the cable from the connection terminals. Improper or loose

connections can cause heat generation or fire.

EN

•

Install dedicated electric outlet and circuit breaker before using the

air conditioner.

•

Do not connect the ground wire to a gas pipe, a lightning rod, or a

telephone ground wire.

Operation

•

Be sure to use only those parts listed in the service parts list. Never

attempt to modify the equipment.

•

Make sure that children do not climb on or hit the outdoor unit.

•

Dispose of the batteries in a place where there is no danger of fire.

•

Use only the refrigerant specified on the air conditioner label.

•

Cut off the power supply if there is any noise, smell, or smoke

coming from the air conditioner.

•

Do not leave flammable substances such as gasoline, benzene, or

thinner near the air conditioner.

•

Contact an authorized service center when the air conditioner is

submerged by flood waters.

•

Do not use the air conditioner for an extended period of time in a

small place without proper ventilation.

•

In the event of a gas leak (such as Freon, propane gas, LP gas,

etc.) ventilate sufficiently before using the air conditioner again.

•

To clean the interior, contact an authorized service center or dealer.

Using harsh detergents may cause corrosion or damage to the unit.

•

Be sure to ventilate sufficiently when the air conditioner and a

heating appliance such as a heater are used simultaneously.

EN

•

Install dedicated electric outlet and circuit breaker before using the

air conditioner.

•

Do not connect the ground wire to a gas pipe, a lightning rod, or a

telephone ground wire.

Operation

•

Be sure to use only those parts listed in the service parts list. Never

attempt to modify the equipment.

•

Make sure that children do not climb on or hit the outdoor unit.

•

Dispose of the batteries in a place where there is no danger of fire.

•

Use only the refrigerant specified on the air conditioner label.

•

Cut off the power supply if there is any noise, smell, or smoke

coming from the air conditioner.

•

Do not leave flammable substances such as gasoline, benzene, or

thinner near the air conditioner.

•

Contact an authorized service center when the air conditioner is

submerged by flood waters.

•

Do not use the air conditioner for an extended period of time in a

small place without proper ventilation.

•

In the event of a gas leak (such as Freon, propane gas, LP gas,

etc.) ventilate sufficiently before using the air conditioner again.

•

To clean the interior, contact an authorized service center or dealer.

Using harsh detergents may cause corrosion or damage to the unit.

•

Be sure to ventilate sufficiently when the air conditioner and a

heating appliance such as a heater are used simultaneously.

EN

•

Do not block the inlet or outlet of air flow.

•

Do not insert hands or other objects through the air inlet or outlet

while the air conditioner is operating.

•

Make sure that the power cable is neither dirty, loose, nor broken.

•

Never touch, operate, or repair the air conditioner with wet hands.

•

Do not place any objects on the power cable.

•

Do not place a heater or other heating appliances near the power

cable.

•

Do not modify or extend the power cable. Scratches or peeling

insulation on the power cables may result in fire or electric shock,

and should be replaced.

•

Cut off the power supply immediately in the event of a blackout or a

thunderstorm.

•

Take care to ensure that power cable could not be pulled out or

damaged during operation.

•

Do not touch refrigerant pipe or water pipe or any internal parts

while the unit is operation or immediately after operation.

Maintenance

•

Do not clean the appliance by spraying water directly onto the

product.

•

Before cleaning or performing maintenance, disconnect the power

supply and wait until the fan stops.

EN

•

Do not block the inlet or outlet of air flow.

•

Do not insert hands or other objects through the air inlet or outlet

while the air conditioner is operating.

•

Make sure that the power cable is neither dirty, loose, nor broken.

•

Never touch, operate, or repair the air conditioner with wet hands.

•

Do not place any objects on the power cable.

•

Do not place a heater or other heating appliances near the power

cable.

•

Do not modify or extend the power cable. Scratches or peeling

insulation on the power cables may result in fire or electric shock,

and should be replaced.

•

Cut off the power supply immediately in the event of a blackout or a

thunderstorm.

•

Take care to ensure that power cable could not be pulled out or

damaged during operation.

•

Do not touch refrigerant pipe or water pipe or any internal parts

while the unit is operation or immediately after operation.

Maintenance

•

Do not clean the appliance by spraying water directly onto the

product.

•

Before cleaning or performing maintenance, disconnect the power

supply and wait until the fan stops.

EN

Technical Safety

•

Installation or repairs made by unauthorized persons can pose

hazards to you and others.

•

The information contained in the manual is intended for use

by a qualified service technician who is familiar with the safety

procedures and equipped with the proper tools and test

instruments.

•

Failure to read and follow all instructions in this manual can result

in equipment malfunction, property damage, personal injury and/or

death.

•

The appliance shall be installed in accordance with national wiring

regulations.

•

When the power cord is to be replaced, the replacement work

shall be performed by authorized personnel using only genuine

replacement parts.

•

This appliance must be properly grounded to minimize risk of

electric shock.

•

Do not cut or remove the grounding prong from the power plug.

•

Attaching the adapter ground terminal to the wall receptacle cover

screw does not ground the appliance unless the cover screw is

metal, uninsulated, and the wall receptacle is grounded through

house wiring.

•

If you have any doubt whether the air conditioner is properly

grounded, have the wall receptacle and circuit checked by a

qualified electrician.

•

The refrigerant and insulation blowing gas used in the appliance

require special disposal procedures. Consult a service agent or a

similarly qualified person before disposing of them.

•

If the supply cord is damaged, it must be replaced by the

manufacturer or its service agents or similarly qualified person in

order to avoid a hazard.

EN

Technical Safety

•

Installation or repairs made by unauthorized persons can pose

hazards to you and others.

•

The information contained in the manual is intended for use

by a qualified service technician who is familiar with the safety

procedures and equipped with the proper tools and test

instruments.

•

Failure to read and follow all instructions in this manual can result

in equipment malfunction, property damage, personal injury and/or

death.

•

The appliance shall be installed in accordance with national wiring

regulations.

•

When the power cord is to be replaced, the replacement work

shall be performed by authorized personnel using only genuine

replacement parts.

•

This appliance must be properly grounded to minimize risk of

electric shock.

•

Do not cut or remove the grounding prong from the power plug.

•

Attaching the adapter ground terminal to the wall receptacle cover

screw does not ground the appliance unless the cover screw is

metal, uninsulated, and the wall receptacle is grounded through

house wiring.

•

If you have any doubt whether the air conditioner is properly

grounded, have the wall receptacle and circuit checked by a

qualified electrician.

•

The refrigerant and insulation blowing gas used in the appliance

require special disposal procedures. Consult a service agent or a

similarly qualified person before disposing of them.

•

If the supply cord is damaged, it must be replaced by the

manufacturer or its service agents or similarly qualified person in

order to avoid a hazard.

EN

CAUTION

To reduce the risk of minor injury to persons, malfunction, or

damage to the product or property when using this product,

follow basic precautions, including the following:

Installation

•

Do not install the air conditioner in an area where it is directly

exposed to sea wind (salt spray).

•

Install the drain hose properly for the smooth drainage of water

condensation.

•

Exercise caution when unpacking or installing the air conditioner.

•

Do not touch the leaking refrigerant during installation or repair.

•

Transport the air conditioner with two or more people or use a

forklift.

•

Install the outdoor unit such that it is protected from direct sunlight.

Do not place the indoor unit in a place where it is directly exposed

to sunlight via the windows.

•

Safely dispose of packing materials such as screws, nails or

batteries using proper packaging after installation or repair.

•

Install the air conditioner in a place where the noise from the

outdoor unit or the exhaust fumes will not inconvenience the

neighbors. Failure to do so may result in conflict with the neighbors.

A

EN

CAUTION

To reduce the risk of minor injury to persons, malfunction, or

damage to the product or property when using this product,

follow basic precautions, including the following:

Installation

•

Do not install the air conditioner in an area where it is directly

exposed to sea wind (salt spray).

•

Install the drain hose properly for the smooth drainage of water

condensation.

•

Exercise caution when unpacking or installing the air conditioner.

•

Do not touch the leaking refrigerant during installation or repair.

•

Transport the air conditioner with two or more people or use a

forklift.

•

Install the outdoor unit such that it is protected from direct sunlight.

Do not place the indoor unit in a place where it is directly exposed

to sunlight via the windows.

•

Safely dispose of packing materials such as screws, nails or

batteries using proper packaging after installation or repair.

•

Install the air conditioner in a place where the noise from the

outdoor unit or the exhaust fumes will not inconvenience the

neighbors. Failure to do so may result in conflict with the neighbors.

A

EN

Operation

•

Remove the batteries if the remote control is not to be used for an

extended period of time.

•

Make sure that the filter is installed before operating the air

conditioner.

•

Be sure to check if there is a refrigerant leak after installing or

repairing the air conditioner

.

•

Do not place any object on the air conditioner.

•

Never mix different types of batteries, or old and new batteries for

the remote control.

•

Do not let the air conditioner run for a long time when humidity is

very high or when a door or a window has been left open.

•

Stop using the remote control if there is a fluid leak in the battery. If

your clothes or skin are exposed to the leaking battery fluid, wash

of

f with clean water.

•

Do not expose people, animals, or plants to the cold or hot wind

from the air conditioner for extended periods of time.

•

If the leaking battery fluid has been swallowed, wash off the inside

of the mouth thoroughly and consult a doctor

.

•

Do not drink the water drained from the air conditioner.

•

Do not use the product for special purposes, such as preserving

foods, works of art, and etc. It is an air conditioner for consumer

purposes, not a precision refrigeration system. There is risk of

damage or loss of property

.

•

Do not recharge or disassemble the batteries.

EN

Operation

•

Remove the batteries if the remote control is not to be used for an

extended period of time.

•

Make sure that the filter is installed before operating the air

conditioner.

•

Be sure to check if there is a refrigerant leak after installing or

repairing the air conditioner

.

•

Do not place any object on the air conditioner.

•

Never mix different types of batteries, or old and new batteries for

the remote control.

•

Do not let the air conditioner run for a long time when humidity is

very high or when a door or a window has been left open.

•

Stop using the remote control if there is a fluid leak in the battery. If

your clothes or skin are exposed to the leaking battery fluid, wash

of

f with clean water.

•

Do not expose people, animals, or plants to the cold or hot wind

from the air conditioner for extended periods of time.

•

If the leaking battery fluid has been swallowed, wash off the inside

of the mouth thoroughly and consult a doctor

.

•

Do not drink the water drained from the air conditioner.

•

Do not use the product for special purposes, such as preserving

foods, works of art, and etc. It is an air conditioner for consumer

purposes, not a precision refrigeration system. There is risk of

damage or loss of property

.

•

Do not recharge or disassemble the batteries.

EN

Maintenance

•

Never touch the metal parts of the air conditioner when removing

the air filter.

•

Use a sturdy stool or ladder when cleaning, maintaining, or

repairing the air conditioner at a height.

•

Never use strong cleaning agents or solvents when cleaning the air

conditioner or spray water. Use a smooth cloth.

I

EN

Maintenance

•

Never touch the metal parts of the air conditioner when removing

the air filter.

•

Use a sturdy stool or ladder when cleaning, maintaining, or

repairing the air conditioner at a height.

•

Never use strong cleaning agents or solvents when cleaning the air

conditioner or spray water. Use a smooth cloth.

I

OPERATION

EN

Notes for Operation

Suggestion for Energy Saving

•

Do not cool excessively indoors. This may be harmful for your health and may consume more

electricity.

•

Block sunlight with blinds or curtains while you are operating the air conditioner.

•

Keep doors or windows closed tightly while you are operating the air conditioner.

•

Adjust the direction of the air flow vertically or horizontally to circulate indoor air.

•

Speed up the fan to cool or warm indoor air quickly, within a short period of time.

•

Open windows regularly for ventilation. The indoor air quality may deteriorate if the air conditioner is

used for long durations.

•

Clean the air filter once every 2 weeks. Dust and impurities collected in the air filter may block the air

flow or weaken the cooling / dehumidifying functions.

•

For use in Australia, New Zealand :

This air conditioner is capable of DRM1, DRM2 and DRM3 and

complies with standard AS/NZS 4755.3.1.(Optional)

Parts and Functions

Indoor Unit

Outdoor Unit

Air Filter

Air Intake

Front Cover

On/Off Button

Air Deflector (Vertical Louver)

Air Deflector (Horizontal Vane)

Air Outlet

1

2

3

4

5

6

7

Air Intake Vents

Air Outlet Vents

1

2

NOTE

•

The number and location of operation lamps may vary according to the model of the air conditioner.

•

The feature may be changed according to the type of model.

□

□

□

□

□

□

□

□

□

OPERATION

EN

Notes for Operation

Suggestion for Energy Saving

•

Do not cool excessively indoors. This may be harmful for your health and may consume more

electricity.

•

Block sunlight with blinds or curtains while you are operating the air conditioner.

•

Keep doors or windows closed tightly while you are operating the air conditioner.

•

Adjust the direction of the air flow vertically or horizontally to circulate indoor air.

•

Speed up the fan to cool or warm indoor air quickly, within a short period of time.

•

Open windows regularly for ventilation. The indoor air quality may deteriorate if the air conditioner is

used for long durations.

•

Clean the air filter once every 2 weeks. Dust and impurities collected in the air filter may block the air

flow or weaken the cooling / dehumidifying functions.

•

For use in Australia, New Zealand :

This air conditioner is capable of DRM1, DRM2 and DRM3 and

complies with standard AS/NZS 4755.3.1.(Optional)

Parts and Functions

Indoor Unit

Outdoor Unit

Air Filter

Air Intake

Front Cover

On/Off Button

Air Deflector (Vertical Louver)

Air Deflector (Horizontal Vane)

Air Outlet

1

2

3

4

5

6

7

Air Intake Vents

Air Outlet Vents

1

2

NOTE

•

The number and location of operation lamps may vary according to the model of the air conditioner.

•

The feature may be changed according to the type of model.

□

□

□

□

□

□

□

□

□

EN

Wireless Remote Control

Inserting Batteries

If the display screen of the remote control begins

to fade, replace the batteries. Insert AAA (1.5 V)

batteries before using the remote control.

1

Remove the battery cover.

2

Insert the new batteries and make sure that

the + and - terminals of the batteries are

installed correctly.

Installing the Remote Control

Holder

To protect the remote control, install the holder

where there is no direct sunlight.

1

Choose a safe and easily accessible

location.

2

Fix the holder by fastening 2 screws firmly

with a screwdriver.

Operating Method

Point the remote control towards the signal

receiver at the right side of the air conditioner to

operate it.

NOTE

•

The remote control may operate other

electronic devices if it is directed towards them.

Make sure to point the remote control towards

the air conditioner signal receiver.

•

For proper operation, use a soft cloth to clean

the signal transmitter and receiver.

•

In case of a function not provided in the

product, no buzzer sound occurs from the

product when a button for such function on the

remote control is pressed except for Air Flow

Direction (

SWING

)

, Energy Display

(

kW [3 s]

)

, Air

Purification function (

).

EN

Wireless Remote Control

Inserting Batteries

If the display screen of the remote control begins

to fade, replace the batteries. Insert AAA (1.5 V)

batteries before using the remote control.

1

Remove the battery cover.

2

Insert the new batteries and make sure that

the + and - terminals of the batteries are

installed correctly.

Installing the Remote Control

Holder

To protect the remote control, install the holder

where there is no direct sunlight.

1

Choose a safe and easily accessible

location.

2

Fix the holder by fastening 2 screws firmly

with a screwdriver.

Operating Method

Point the remote control towards the signal

receiver at the right side of the air conditioner to

operate it.

NOTE

•

The remote control may operate other

electronic devices if it is directed towards them.

Make sure to point the remote control towards

the air conditioner signal receiver.

•

For proper operation, use a soft cloth to clean

the signal transmitter and receiver.

•

In case of a function not provided in the

product, no buzzer sound occurs from the

product when a button for such function on the

remote control is pressed except for Air Flow

Direction (

SWING

)

, Energy Display

(

kW [3 s]

)

, Air

Purification function (

).

EN

Setting the Current Time

1

Insert batteries.

•

The icon below blinks at the bottom of the

display screen.

2

Press

or

button to select the

minutes.

3

Press

SET/CANCEL

button to finish.

NOTE

•

The On/Off Timer is available after setting the

current time.

Using the °C/°F Conversion

Function (Optional)

This function changes unit between °C and °F

•

Press and hold

℃↔℉ [5 s]

SWING

button about 5 seconds.

Operating the Air Conditioner

without the Remote Control

You can use the

ON/OFF

button of the indoor

unit to operate the air conditioner when the

remote control is unavailable.

1

Open the front cover (Type2) or horizontal

vane (Type1).

2

Press the

ON/OFF

button.

Type1

ON/OFF

Type2

ON/OFF

NOTE

•

The stepping motor may be broken, if the

horizontal vane opens rapidly.

•

The fan speed is set to high.

•

The feature may be changed according to the

type of model.

•

The temperature cannot be altered when using

this emergency On/Off button.

•

For cooling-only models, the temperature is set

to 22 °C

•

For cooling & heating models, the temperature

is set from 22 °C to 24 °C

EN

Using Wireless Remote Control

You can operate the air conditioner more conveniently with the remote control.

SWING

kW [3 s]

℃↔℉ [5 s]

DIAGNOSIS [5 s]

SWING

SET UP

ROOM

TEMP

JET

MODE

MODE

TEMP

FAN

SPEED

FUNC.

TIMER

CANCEL

SET

CANCEL

*

*

1

2

*

RESET

Button

Display

Screen

Description

-

To turn on/off the air conditioner.

To adjust the desired room temperature

in cooling, heating or auto changeover

mode.

MODE

To select the cooling mode.

To select the heating mode.

To select the dehumidification mode.

To select the fan mode.

To select the auto changeover/auto

operation mode.

JET

MODE

To change room temperature quickly.

FAN

SPEED

To adjust the fan speed.

SWING

SWING

To adjust the air flow direction vertically

or horizontally.

NOTE

•

* buttons may be changed according to the type of model.

00

ti

-

-

,,,

6

Po

EN

Using Wireless Remote Control

You can operate the air conditioner more conveniently with the remote control.

SWING

kW [3 s]

℃↔℉ [5 s]

DIAGNOSIS [5 s]

SWING

SET UP

ROOM

TEMP

JET

MODE

MODE

TEMP

FAN

SPEED

FUNC.

TIMER

CANCEL

SET

CANCEL

*

*

1

2

*

RESET

Button

Display

Screen

Description

-

To turn on/off the air conditioner.

To adjust the desired room temperature

in cooling, heating or auto changeover

mode.

MODE

To select the cooling mode.

To select the heating mode.

To select the dehumidification mode.

To select the fan mode.

To select the auto changeover/auto

operation mode.

JET

MODE

To change room temperature quickly.

FAN

SPEED

To adjust the fan speed.

SWING

SWING

To adjust the air flow direction vertically

or horizontally.

NOTE

•

* buttons may be changed according to the type of model.

00

ti

-

-

,,,

6

Po

EN

SWING

kW [3 s]

℃↔℉ [5 s]

DIAGNOSIS [5 s]

SWING

SET UP

ROOM

TEMP

JET

MODE

MODE

TEMP

FAN

SPEED

FUNC.

TIMER

CANCEL

SET

CANCEL

*

*

1

2

*

RESET

Button

Display

Screen

Description

TIMER

To turn on/off air conditioner

automatically at desired time.

SET/

CANCEL

-

To set/cancel the special functions and

timer.

CANCEL

-

To cancel the timer settings.

-

To adjust time.

*LIGHT

OFF

-

To set the brightness of the display on

the indoor unit.

ROOM

TEMP

To display the room temperature.

°C↔°F [5 s]

To change unit between °C and °F

*ENERGY

SAVING

To minimize power consumption.

*COMFORT

AIR

To adjust the air flow to deflect wind.

kW [3 s]

-

To set whether or not to display

information regarding energy.

*ENERGY

CTRL

To bring the effect of the power saving.

*COMFORT

SLEEP

To make a comfortable sleeping

environment.

DIAGNOSIS

]

[5

s

-

To conveniently check maintenance

information of a product.

D

[

00

C!l

~[§]~~

~

GEJ

@

J

oc

Of

~

~7]

l88"

9

~

EN

SWING

kW [3 s]

℃↔℉ [5 s]

DIAGNOSIS [5 s]

SWING

SET UP

ROOM

TEMP

JET

MODE

MODE

TEMP

FAN

SPEED

FUNC.

TIMER

CANCEL

SET

CANCEL

*

*

1

2

*

RESET

Button

Display

Screen

Description

TIMER

To turn on/off air conditioner

automatically at desired time.

SET/

CANCEL

-

To set/cancel the special functions and

timer.

CANCEL

-

To cancel the timer settings.

-

To adjust time.

*LIGHT

OFF

-

To set the brightness of the display on

the indoor unit.

ROOM

TEMP

To display the room temperature.

°C↔°F [5 s]

To change unit between °C and °F

*ENERGY

SAVING

To minimize power consumption.

*COMFORT

AIR

To adjust the air flow to deflect wind.

kW [3 s]

-

To set whether or not to display

information regarding energy.

*ENERGY

CTRL

To bring the effect of the power saving.

*COMFORT

SLEEP

To make a comfortable sleeping

environment.

DIAGNOSIS

]

[5

s

-

To conveniently check maintenance

information of a product.

D

[

00

C!l

~[§]~~

~

GEJ

@

J

oc

Of

~

~7]

l88"

9

~

EN

SWING

kW [3 s]

℃↔℉ [5 s]

DIAGNOSIS [5 s]

SWING

SET UP

ROOM

TEMP

JET

MODE

MODE

TEMP

FAN

SPEED

FUNC.

TIMER

CANCEL

SET

CANCEL

*

*

1

2

*

RESET

Button

Display

Screen

Description

FUNC.

To purify the air by removing particles

that enters the indoor unit.

To reduce noise from outdoor units.

To keep your skin moisturized by

generating ion clusters.

To lower indoor humidity quickly.

To maintain a minimum room

temperature and prevent objects in the

room from freezing.

To scare away a mosquito.

To remove moisture generated inside

the indoor unit.

To make a comfortable sleeping

environment.

RESET

-

To Initialize the remote control settings.

NOTE

•

Some functions may not be supported, depending on the model.

•

* buttons may be changed according to the type of model.

•

Press the

SET/CANCEL

button to operate the selected

FUNC

.

'

'

c~~=

~68

~~~)

[

D

(!]

00

~

~

~

mlPo

1!11

LH

m

~

~SL

EN

SWING

kW [3 s]

℃↔℉ [5 s]

DIAGNOSIS [5 s]

SWING

SET UP

ROOM

TEMP

JET

MODE

MODE

TEMP

FAN

SPEED

FUNC.

TIMER

CANCEL

SET

CANCEL

*

*

1

2

*

RESET

Button

Display

Screen

Description

FUNC.

To purify the air by removing particles

that enters the indoor unit.

To reduce noise from outdoor units.

To keep your skin moisturized by

generating ion clusters.

To lower indoor humidity quickly.

To maintain a minimum room

temperature and prevent objects in the

room from freezing.

To scare away a mosquito.

To remove moisture generated inside

the indoor unit.

To make a comfortable sleeping

environment.

RESET

-

To Initialize the remote control settings.

NOTE

•

Some functions may not be supported, depending on the model.

•

* buttons may be changed according to the type of model.

•

Press the

SET/CANCEL

button to operate the selected

FUNC

.

'

'

c~~=

~68

~~~)

[

D

(!]

00

~

~

~

mlPo

1!11

LH

m

~

~SL

EN

Restarting the Air Conditioner

Automatically

When the air conditioner is turned on again

after a power failure, this function restores the

previous settings.

Disabling Auto Restart

1

Open the front cover (Type2) or horizontal

vane (Type1).

2

Press the

ON/OFF

button and hold it for 6

seconds, then the unit will beep twice and

the lamp will blink twice 4 times.

•

To re-enable the function, press the

ON/

OFF

button and hold it for 6 seconds. The

unit will beep twice and the lamp will blink

times.

4

Type1

ON/OFF

Type2

ON/OFF

NOTE

•

The feature may be changed according to the

type of model.

•

If you press and hold the

ON/OFF

button for

3

– 5 seconds, instead of 6 seconds, the unit

will switch to the test operation. In the test

operation, the unit blows out strong cooling

air for 18 minutes and then returns to factory

default settings.

Using the Mode Function

This function allows you to select the desired

function.

Cooling Only Model

Cooling Mode

Auto Operation(AI) Mode

Dehumidification Mode

Fan Mode

Cooling & Heating Model

Cooling Mode

Auto Changeover Mode

Dehumidification Mode

Heating Mode

Fan Mode

Cooling Mode

1

Turn the appliance on.

2

Press

MODE

button repeatedly to select the

Cooling Mode.

•

is displayed on the display screen.

3

Press

or

button to set the desired

temperature.

I

~~

I

I

•

I

@I

I

•

I

6

I

I

•

I

~

I

I

I

I

I

~~

•

I

~

@I

I

•

I

6

I

I

•

I

I

:-

¢

-:

I

~

•

I

{

i:

___

I

EN

Restarting the Air Conditioner

Automatically

When the air conditioner is turned on again

after a power failure, this function restores the

previous settings.

Disabling Auto Restart

1

Open the front cover (Type2) or horizontal

vane (Type1).

2

Press the

ON/OFF

button and hold it for 6

seconds, then the unit will beep twice and

the lamp will blink twice 4 times.

•

To re-enable the function, press the

ON/

OFF

button and hold it for 6 seconds. The

unit will beep twice and the lamp will blink

times.

4

Type1

ON/OFF

Type2

ON/OFF

NOTE

•

The feature may be changed according to the

type of model.

•

If you press and hold the

ON/OFF

button for

3

– 5 seconds, instead of 6 seconds, the unit

will switch to the test operation. In the test

operation, the unit blows out strong cooling

air for 18 minutes and then returns to factory

default settings.

Using the Mode Function

This function allows you to select the desired

function.

Cooling Only Model

Cooling Mode

Auto Operation(AI) Mode

Dehumidification Mode

Fan Mode

Cooling & Heating Model

Cooling Mode

Auto Changeover Mode

Dehumidification Mode

Heating Mode

Fan Mode

Cooling Mode

1

Turn the appliance on.

2

Press

MODE

button repeatedly to select the

Cooling Mode.

•

is displayed on the display screen.

3

Press

or

button to set the desired

temperature.

I

~~

I

I

•

I

@I

I

•

I

6

I

I

•

I

~

I

I

I

I

I

~~

•

I

~

@I

I

•

I

6

I

I

•

I

I

:-

¢

-:

I

~

•

I

{

i:

___

I

EN

Heating Mode

1

Turn the appliance on.

2

Press

MODE

button repeatedly to select the

Heating Mode.

•

is displayed on the display screen.

3

Press

or

button to set the desired

temperature.

Dehumidification Mode

This mode removes excess moisture from a

highly humid environment or during the rainy

season, in order to prevent molding from setting

in. This mode adjusts the room temperature

and the fan speed automatically to maintain the

optimal level of humidity.

1

Turn the appliance on.

2

Press

MODE

button repeatedly to select the

Dehumidification Mode.

•

is displayed on the display screen.

NOTE

•

In this mode you cannot adjust the room

temperature, it is adjusted automatically.

•

The room temperature is not displayed on the

display screen.

Fan Mode

This mode only circulates the indoor air without

changing the room temperature.

1

Turn the appliance on.

2

Press

MODE

button repeatedly to select the

Fan Mode.

•

is displayed on the display screen.

3

Press

FAN SPEED

button to adjust the fan

speed.

Auto Changeover Mode

Cooling & Heating Model

This mode changes the mode automatically to

maintain the set temperature at ±2 °C

1

Turn the appliance on.

2

Press

MODE

button repeatedly to select the

Auto Changeover Mode.

•

is displayed on the display screen.

3

Press

or

button to set the desired

temperature.

4

Press

FAN SPEED

button to adjust the fan

speed.

-

ti.

-

,''

00

00

EN

Heating Mode

1

Turn the appliance on.

2

Press

MODE

button repeatedly to select the

Heating Mode.

•

is displayed on the display screen.

3

Press

or

button to set the desired

temperature.

Dehumidification Mode

This mode removes excess moisture from a

highly humid environment or during the rainy

season, in order to prevent molding from setting

in. This mode adjusts the room temperature

and the fan speed automatically to maintain the

optimal level of humidity.

1

Turn the appliance on.

2

Press

MODE

button repeatedly to select the

Dehumidification Mode.

•

is displayed on the display screen.

NOTE

•

In this mode you cannot adjust the room

temperature, it is adjusted automatically.

•

The room temperature is not displayed on the

display screen.

Fan Mode

This mode only circulates the indoor air without

changing the room temperature.

1

Turn the appliance on.

2

Press

MODE

button repeatedly to select the

Fan Mode.

•

is displayed on the display screen.

3

Press

FAN SPEED

button to adjust the fan

speed.

Auto Changeover Mode

Cooling & Heating Model

This mode changes the mode automatically to

maintain the set temperature at ±2 °C

1

Turn the appliance on.

2

Press

MODE

button repeatedly to select the

Auto Changeover Mode.

•

is displayed on the display screen.

3

Press

or

button to set the desired

temperature.

4

Press

FAN SPEED

button to adjust the fan

speed.

-

ti.

-

,''

00

00

EN

Auto Operation (Artificial

Intelligence)

Cooling Only Model

This mode automatically sets the fan speed

and temperature depending on the room

temperature.

1

Turn the appliance on.

2

Press

MODE

button repeatedly to select the

Auto Operation.

•

is displayed on the display screen.

3

Press

or

button to select the desired

operation code if the temperature is higher

or lower than the desired temperature.

Code

Description

Cold

2

1

A little cold

0

Maintain room temperature

-1

A little hot

Hot

-2

NOTE

•

In this mode, you cannot adjust the fan speed,

but you can set the air deflector to rotate

automatically.

Using the Jet Mode Function

Changing Room Temperature

Quickly

This function allows you to cool indoor air quickly

during the summer or warm it quickly during

winter.

•

Cooling & Heating model : The Jet Mode

function is available with Cooling, Heating and

Dehumidification Mode.

•

Cooling Only model : The Jet Mode function

is available with Cooling, Dehumidification,

Fan,

Auto Operation Mode and Air Purification

function.

1

Turn the appliance on.

2

Press

MODE

button repeatedly to select the

desired mode.

3

Press

JET MODE

button.

•

is displayed on the display screen.

NOTE

•

Jet Heating Mode is not available on some

models.

•

In Jet Cooling Mode, strong air blows out for

30

minutes.

•

After 30 minutes later, setting temperature is

maintaining 18 °C

If you want to change the temperature,

Press

or

button to set the desired

temperature.

•

In Jet Heating Mode, strong air blows out for

30

minutes.

•

After 30 minutes later, setting temperature is

maintaining 30 °C

If you want to change the temperature,

Press

or

button to set the desired

temperature.

•

This function may operate differently from the

remote control display

.

00

2

Po

L

j

~

-

l

~

~

(

~

➔

(

"

I

J

00

00

EN

Auto Operation (Artificial

Intelligence)

Cooling Only Model

This mode automatically sets the fan speed

and temperature depending on the room

temperature.

1

Turn the appliance on.

2

Press

MODE

button repeatedly to select the

Auto Operation.

•

is displayed on the display screen.

3

Press

or

button to select the desired

operation code if the temperature is higher

or lower than the desired temperature.

Code

Description

Cold

2

1

A little cold

0

Maintain room temperature

-1

A little hot

Hot

-2

NOTE

•

In this mode, you cannot adjust the fan speed,

but you can set the air deflector to rotate

automatically.

Using the Jet Mode Function

Changing Room Temperature

Quickly

This function allows you to cool indoor air quickly

during the summer or warm it quickly during

winter.

•

Cooling & Heating model : The Jet Mode

function is available with Cooling, Heating and

Dehumidification Mode.

•

Cooling Only model : The Jet Mode function

is available with Cooling, Dehumidification,

Fan,

Auto Operation Mode and Air Purification

function.

1

Turn the appliance on.

2

Press

MODE

button repeatedly to select the

desired mode.

3

Press

JET MODE

button.

•

is displayed on the display screen.

NOTE

•

Jet Heating Mode is not available on some

models.

•

In Jet Cooling Mode, strong air blows out for

30

minutes.

•

After 30 minutes later, setting temperature is

maintaining 18 °C

If you want to change the temperature,

Press

or

button to set the desired

temperature.

•

In Jet Heating Mode, strong air blows out for

30

minutes.

•

After 30 minutes later, setting temperature is

maintaining 30 °C

If you want to change the temperature,

Press

or

button to set the desired

temperature.

•

This function may operate differently from the

remote control display

.

00

2

Po

L

j

~

-

l

~

~

(

~

➔

(

"

I

J

00

00

EN

Using the Fan Speed Function

Adjusting the Fan Speed

•

Press

FAN SPEED

button repeatedly to adjust

the fan speed.

Display Screen

Speed

High

Medium - High

Medium

Medium - Low

Low

-

Natural Wind

NOTE

•

The fan speed of Natural Wind adjusts

automatically.

→

→

•

The fan speed icons are displayed on some

indoor units.

→

→

→

→

•

Indoor unit display is displayed only 5 seconds

and return to the setting temperature on some

models.

Using the Air Flow Direction

Function

This function adjusts the direction of the air flow

vertically (horizontally).

•

Press

SWING

(

SWING

)

button repeatedly and select

the desired direction.

−

Select

(

to adjust the direction of the

)

air flow automatically.

NOTE

•

Adjusting the air flow direction horizontally may

not be supported, depending on the model.

•

Adjusting the air deflector arbitrarily may cause

product failure.

•

If you restart the air conditioner, it starts to

operate with the previously set direction of air

flow

, so the air deflector may not match the

icon displayed on the remote control. When

this occurs, press

SWING

or

SWING

button to adjust

the direction of the air flow again.

•

This function may operate differently from the

remote control display.

-

-

-

-

-

-

-

=

--

FI

F2

F3

F~

FS

EN

Using the Fan Speed Function

Adjusting the Fan Speed

•

Press

FAN SPEED

button repeatedly to adjust

the fan speed.

Display Screen

Speed

High

Medium - High

Medium

Medium - Low

Low

-

Natural Wind

NOTE

•

The fan speed of Natural Wind adjusts

automatically.

→

→

•

The fan speed icons are displayed on some

indoor units.

→

→

→

→

•

Indoor unit display is displayed only 5 seconds

and return to the setting temperature on some

models.

Using the Air Flow Direction

Function

This function adjusts the direction of the air flow

vertically (horizontally).

•

Press

SWING

(

SWING

)

button repeatedly and select

the desired direction.

−

Select

(

to adjust the direction of the

)

air flow automatically.

NOTE

•

Adjusting the air flow direction horizontally may

not be supported, depending on the model.

•

Adjusting the air deflector arbitrarily may cause

product failure.

•

If you restart the air conditioner, it starts to

operate with the previously set direction of air

flow

, so the air deflector may not match the

icon displayed on the remote control. When

this occurs, press

SWING

or

SWING

button to adjust

the direction of the air flow again.

•

This function may operate differently from the

remote control display.

-

-

-

-

-

-

-

=

--

FI

F2

F3

F~

FS

EN

Setting the On/Off Timer

This function sets the air conditioner to turn on/

off automatically at desired time.

Setting the On Timer

1

Press

TIMER

button repeatedly.

•

The icon below blinks at the bottom of the

display screen.

2

Press

or

button to select the

minutes.

3

Press

SET/CANCEL

button to finish.

•

After setting the timer, current time and

icon are displayed on the display screen

indicating that the desired time is set.

Canceling the On Timer

1

Press

TIMER

button repeatedly.

•

The icon below blinks at the bottom of the

display screen.

2

Press

SET/CANCEL

button to cancel the

setting.

Setting the Off Timer

1

Press

TIMER

button repeatedly.

•

The icon below blinks at the bottom of the

display screen.

2

Press

or

button to select the

minutes.

3

Press

SET/CANCEL

button to finish.

•

After setting the timer, current time and

icon are displayed on the display screen

indicating that the desired time is set.

NOTE

•

This function is disabled when you set Simple

Timer.

Canceling the Off Timer

1

Press

TIMER

button repeatedly.

•

The icon below blinks at the bottom of the

display screen.

2

Press

SET/CANCEL

button to cancel the

setting.

Canceling the Timer Setting

•

Press

CANCEL

button to cancel all timer settings.

~

EN

Setting the On/Off Timer

This function sets the air conditioner to turn on/

off automatically at desired time.

Setting the On Timer

1

Press

TIMER

button repeatedly.

•

The icon below blinks at the bottom of the

display screen.

2

Press

or

button to select the

minutes.

3

Press

SET/CANCEL

button to finish.

•

After setting the timer, current time and

icon are displayed on the display screen

indicating that the desired time is set.

Canceling the On Timer

1

Press

TIMER

button repeatedly.

•

The icon below blinks at the bottom of the

display screen.

2

Press

SET/CANCEL

button to cancel the

setting.

Setting the Off Timer

1

Press

TIMER

button repeatedly.

•

The icon below blinks at the bottom of the

display screen.

2

Press

or

button to select the

minutes.

3

Press

SET/CANCEL

button to finish.

•

After setting the timer, current time and

icon are displayed on the display screen

indicating that the desired time is set.

NOTE

•

This function is disabled when you set Simple

Timer.

Canceling the Off Timer

1

Press

TIMER

button repeatedly.

•

The icon below blinks at the bottom of the

display screen.

2

Press

SET/CANCEL

button to cancel the

setting.

Canceling the Timer Setting

•

Press

CANCEL

button to cancel all timer settings.

~

EN

Using the Sleep Function

(

Optional

)

This function turns off the air conditioner

automatically when you go to sleep.

1

Turn the appliance on.

2

Press

TIMER

button repeatedly.

•

The icon below blinks at the bottom of the

display screen.

3

Press

or

button to select the hour

(

up to 7 hours

).

4

Press

SET/CANCEL

button to finish.

•

is displayed on the display screen.

NOTE

•

is displayed on some indoor units.

•

Indoor unit display is displayed from 1H to 7H

which is displayed only 5 seconds and then

return to the setting temperature.

•

In the Cooling and Dehumidification Mode, the

temperature increases by 1 °C after 30 minutes

and additional 1 °C after another 30 minutes

for a more comfortable sleep.

•

The temperature increases up to 2 °C from the

preset temperature.

•

Although the remark for fan speed on the

display screen may be changed, the fan speed

is adjusted automatically.

Using the Simple Timer

Function (Optional)

This function turns off the air conditioner

automatically when you go to sleep.

1

Turn the appliance on.

2

Press

TIMER

button repeatedly.

•

The icon below blinks at the bottom of the

display screen.

3

Press

or

button to select the hour

).

(

up to 7 hours

4

Press

SET/CANCEL

button to finish.

•

is displayed on the display screen.

NOTE

•

This function is disabled when you set Off

Timer.

00

IH

EN

Using the Sleep Function

(

Optional

)

This function turns off the air conditioner

automatically when you go to sleep.

1

Turn the appliance on.

2

Press

TIMER

button repeatedly.

•

The icon below blinks at the bottom of the

display screen.

3

Press

or

button to select the hour

(

up to 7 hours

).

4

Press

SET/CANCEL

button to finish.

•

is displayed on the display screen.

NOTE

•

is displayed on some indoor units.

•

Indoor unit display is displayed from 1H to 7H

which is displayed only 5 seconds and then

return to the setting temperature.

•

In the Cooling and Dehumidification Mode, the

temperature increases by 1 °C after 30 minutes

and additional 1 °C after another 30 minutes

for a more comfortable sleep.

•

The temperature increases up to 2 °C from the

preset temperature.

•

Although the remark for fan speed on the

display screen may be changed, the fan speed

is adjusted automatically.

Using the Simple Timer

Function (Optional)

This function turns off the air conditioner

automatically when you go to sleep.

1

Turn the appliance on.

2

Press

TIMER

button repeatedly.

•

The icon below blinks at the bottom of the

display screen.

3

Press

or

button to select the hour

).

(

up to 7 hours

4

Press

SET/CANCEL

button to finish.

•

is displayed on the display screen.

NOTE

•

This function is disabled when you set Off

Timer.

00

IH

EN

Using the Energy Display

Function (Optional)

This function displays the amount of electricity

generated on the indoor display as the product

operates.

Display the Current Energy Usage

1

Turn the appliance on.

2

Press

kW [3 s]

button and hold it about 3

seconds.

•

The instantaneous power consumption (

or

kW

kWh

is displayed for a while on

)

some indoor units.

Display the Cumulative Energy

Usage (Optional)

1

Turn the appliance on.

2

Press

kW [3 s]

button and hold it about 3

seconds twice.

•

The cumulative power consumption (

kW

kWh

)

is displayed for a while on some indoor

units.

NOTE

•

It is not displayed on the wireless remote

control.

•

kW

refers to instantaneous power

consumption.

•

kWh

refers to cumulative power consumption.

•

If it is more than 99 kWh, which is the range of

expression, maintain it as 99 kWh.

•

Power off resets its value.

•

Display wattage under 10 kW(h) with 0.1 kW(h)

unit, and display one over 10 kW(h) with 1

kW(h) unit.

•

The actual power may differ from the power

displayed.

Using the Light Off Function

)

(

Optional

Display Screen Brightness

You can set the brightness of the indoor unit

display screen.

•

Press

LIGHT OFF

button.

NOTE

•

Turn on/off the Display screen.

Using the Comfort Air

Function (Optional)

Comfort Vane Operation

This function conveniently sets the vane to a

preset position to deflect the supply air away

from blowing directly onto the occupants in the

room.

1

Turn the appliance on.

2

Press

COMFORT AIR

button repeatedly and

select the desired direction.

•

or

is displayed on the display

screen.

NOTE

•

or

is displayed on some indoor units.

•

This function is disabled when you press

MODE

or

JET MODE

button.

•

This function is disabled and auto swing of

vertical direction is set up when you press

℃↔℉ [5 s]

SWING

button.

•

When this function is off, the horizontal vane

operates automatically depending on the set

mode.

0

□3

88

'

0

88

.

~

71

l

7

lL,

EN

Using the Energy Display

Function (Optional)

This function displays the amount of electricity

generated on the indoor display as the product

operates.

Display the Current Energy Usage

1

Turn the appliance on.

2

Press

kW [3 s]

button and hold it about 3

seconds.

•

The instantaneous power consumption (

or

kW

kWh

is displayed for a while on

)

some indoor units.

Display the Cumulative Energy

Usage (Optional)

1

Turn the appliance on.

2

Press

kW [3 s]

button and hold it about 3

seconds twice.

•

The cumulative power consumption (

kW

kWh

)

is displayed for a while on some indoor

units.

NOTE

•

It is not displayed on the wireless remote

control.

•

kW

refers to instantaneous power

consumption.

•

kWh

refers to cumulative power consumption.

•

If it is more than 99 kWh, which is the range of

expression, maintain it as 99 kWh.

•

Power off resets its value.

•

Display wattage under 10 kW(h) with 0.1 kW(h)

unit, and display one over 10 kW(h) with 1

kW(h) unit.

•

The actual power may differ from the power

displayed.

Using the Light Off Function

)

(

Optional

Display Screen Brightness

You can set the brightness of the indoor unit

display screen.

•

Press

LIGHT OFF

button.

NOTE

•

Turn on/off the Display screen.

Using the Comfort Air

Function (Optional)

Comfort Vane Operation

This function conveniently sets the vane to a

preset position to deflect the supply air away

from blowing directly onto the occupants in the

room.

1

Turn the appliance on.

2

Press

COMFORT AIR

button repeatedly and

select the desired direction.

•

or

is displayed on the display

screen.

NOTE

•

or

is displayed on some indoor units.

•

This function is disabled when you press

MODE

or

JET MODE

button.

•

This function is disabled and auto swing of

vertical direction is set up when you press

℃↔℉ [5 s]

SWING

button.

•

When this function is off, the horizontal vane

operates automatically depending on the set

mode.

0

□3

88

'

0

88

.

~

71

l

7

lL,

EN

Using the Energy Saving

Function (Optional)

This function minimizes power consumption

during Cooling and increases the set

temperature to the optimal level for a more

comfortable environment. The temperature

automatically settles at 22 °C if room ambient

temperature is below 22 °C The temperature

remains constant if it is above 22 °C

1

Turn the appliance on.

2

Press

MODE

button repeatedly to select the

Cooling Mode.

3

Press

ENERGY SAVING

button.

•

is displayed on the display screen.

Using the Energy Control

Function (Optional)

1

Turn the appliance on.

2

Press

ENERGY CTRL

button.

•

Press

ENERGY CTRL

button repeatedly to

select each step.

NOTE

•

1

step : The power Input is decreased by 20 %

compared to rated power input.

−

is displayed on the display screen.

•

2

step : The power Input is decreased by 40 %

compared to rated power input.

−

is displayed on the display screen.

•

3

step (Optional) : The power Input is

decreased by 60 % compared to rated power

input.

−

is displayed on the display screen.

•

(1 step),

(2 step) is displayed on some

indoor units.

•

This function is available with Cooling Mode.

•

Capacity may decrease when Energy Control

mode is selected.

•

Desired temperature is displayed about 5

seconds if you press

F

AN SPEED

,

or

button.

•

Room temperature is displayed about 5

seconds if you press

ROOM TEMP

button.

00

EN

Using the Energy Saving

Function (Optional)

This function minimizes power consumption

during Cooling and increases the set

temperature to the optimal level for a more

comfortable environment. The temperature

automatically settles at 22 °C if room ambient

temperature is below 22 °C The temperature

remains constant if it is above 22 °C

1

Turn the appliance on.

2

Press

MODE

button repeatedly to select the

Cooling Mode.

3

Press

ENERGY SAVING

button.

•

is displayed on the display screen.

Using the Energy Control

Function (Optional)

1

Turn the appliance on.

2

Press

ENERGY CTRL

button.

•

Press

ENERGY CTRL

button repeatedly to

select each step.

NOTE

•

1

step : The power Input is decreased by 20 %

compared to rated power input.

−

is displayed on the display screen.

•

2

step : The power Input is decreased by 40 %

compared to rated power input.

−

is displayed on the display screen.

•

3

step (Optional) : The power Input is

decreased by 60 % compared to rated power

input.

−

is displayed on the display screen.

•

(1 step),

(2 step) is displayed on some

indoor units.

•

This function is available with Cooling Mode.

•

Capacity may decrease when Energy Control

mode is selected.

•

Desired temperature is displayed about 5

seconds if you press

F

AN SPEED

,

or

button.

•

Room temperature is displayed about 5

seconds if you press

ROOM TEMP

button.

00

EN

Using Special Functions

1

Turn the appliance on.

2

Press

FUNC

button repeatedly to select the

desired function.

3

Press

SET/CANCEL

button to finish.

Display

Screen

Description

To purify the air by removing

particles that enters the indoor

unit.

To reduce noise from outdoor

units.

To keep your skin moisturized

by generating ion clusters.

To lower indoor humidity quickly.

To maintain a minimum room

temperature and prevent objects

in the room from freezing.

To scare away a mosquito.

To remove moisture generated

inside the indoor unit.

To make a comfortable sleeping

environment.

NOTE

•

Some functions may not be supported,

depending on the model.

•

Some functions may operate differently from

the remote control display.

~

~

ll

mlPo

88

IDllH

lj

~

~SL

EN

Using Special Functions

1

Turn the appliance on.

2

Press

FUNC

button repeatedly to select the

desired function.

3

Press

SET/CANCEL

button to finish.

Display

Screen

Description

To purify the air by removing

particles that enters the indoor

unit.

To reduce noise from outdoor

units.

To keep your skin moisturized

by generating ion clusters.

To lower indoor humidity quickly.

To maintain a minimum room

temperature and prevent objects

in the room from freezing.

To scare away a mosquito.

To remove moisture generated

inside the indoor unit.

To make a comfortable sleeping

environment.

NOTE

•

Some functions may not be supported,

depending on the model.

•

Some functions may operate differently from

the remote control display.

~

~

ll

mlPo

88

IDllH

lj

~

~SL

EN

Canceling Special Functions

1

Press

FUNC

button repeatedly to select the

desired function.

2

Press

SET/CANCEL

button to cancel the

function.

Using the Air Purification Function

These functions supply clean and fresh air by

ion particles and the filter.

Description

Display

Function

Ionizer

Ion particles from

Ionizer sterilize

the aerial

bacteria and

other harmful

substances.

Plasma

It removes

microscopic

contaminants

from the intake

air completely to

supply clean and

fresh air.

NOTE

•

You can use this function without turning the air

conditioner on.

•

Both the plasma lamp and the cooling lamp

light up while Smoke A

way/Plasma is in

operation on some models.

•

Do not touch ionizer during operation.

•

This function may operate differently from the

remote control display

.

Using the Silent Function

This function prevents potential claims by

neighbors, reducing noise from outdoor units.

•

is displayed on the display screen.

NOTE

•

This function is disabled when you press

MODE

or

ENERGY CTRL

or

JET MODE

button.

•

This function is available with Cooling, Heating,

Auto Changeover, Auto Operation Mode.

Using the Ion Care Function

This function keeps your skin moisturized by

generating ion clusters which is absorbed into

the surface of skin.

•

is displayed on the display screen about 3

seconds.

NOTE

•

This function is available with Cooling, Fan

Mode and Air Purification function.

Using the Jet Dry Function

This function maximizes the performance of

dehumidification.

•

,

are displayed on the display screen.

NOTE

•

This function is available with Cooling,

Dehumidification, Auto Operation, Fan Mode

and Air Purification function.

•

This function is not available with Sleep

function.

•

This function is disabled when you press

MODE

button.

~

~

ml

Po

EN

Canceling Special Functions

1

Press

FUNC

button repeatedly to select the

desired function.

2

Press

SET/CANCEL

button to cancel the

function.

Using the Air Purification Function

These functions supply clean and fresh air by

ion particles and the filter.

Description

Display

Function

Ionizer

Ion particles from

Ionizer sterilize

the aerial

bacteria and

other harmful

substances.

Plasma

It removes

microscopic

contaminants

from the intake

air completely to

supply clean and

fresh air.

NOTE

•

You can use this function without turning the air

conditioner on.

•

Both the plasma lamp and the cooling lamp

light up while Smoke A

way/Plasma is in

operation on some models.

•

Do not touch ionizer during operation.

•

This function may operate differently from the

remote control display

.

Using the Silent Function

This function prevents potential claims by

neighbors, reducing noise from outdoor units.

•

is displayed on the display screen.

NOTE

•

This function is disabled when you press

MODE

or

ENERGY CTRL

or

JET MODE

button.

•

This function is available with Cooling, Heating,

Auto Changeover, Auto Operation Mode.

Using the Ion Care Function

This function keeps your skin moisturized by

generating ion clusters which is absorbed into

the surface of skin.

•

is displayed on the display screen about 3

seconds.

NOTE

•

This function is available with Cooling, Fan

Mode and Air Purification function.

Using the Jet Dry Function

This function maximizes the performance of

dehumidification.

•

,

are displayed on the display screen.

NOTE

•

This function is available with Cooling,

Dehumidification, Auto Operation, Fan Mode

and Air Purification function.

•

This function is not available with Sleep

function.

•

This function is disabled when you press

MODE

button.

~

~

ml

Po

EN

Using the Low Heating Function

This function operates the heating system to

maintain a minimum room temperature and

prevent objects in the room from freezing where

there is no permanent resident, such as a

vacation.

•

,

are displayed on the display screen.

NOTE

•

This function is available with Heating Mode.

•

During LH operation, when you press buttons

such as

,

,

MODE

,

FAN SPEED

, and

then return to Heating Mode, 30 °C high speed.

•

If

JET MODE

button is pressed during LH

operation, this function is disabled and Power

Heating Mode is operated immediately. (only

for power heating model)

•

In case an error occurred, operation can be

interrupted for the protection of the product.

•

COMFORT AIR

and

SWING

button cannot be

used while the LH function is in operation.

Using the Mosquito Away

Function

This function fans away mosquitos by generating

a high-frequency wave.

•

is displayed on the display screen.

NOTE

•

You can use this function without turning the air

conditioner on.

Using the Auto Clean Function

In the Cooling and Dehumidification Mode,

moisture is generated inside the indoor unit. This

function removes such moisture.

•

is displayed on the display screen.

NOTE

•

Some functions cannot be used while the Auto

Clean function is in operation.

•

If you turn off the power, the fan operates for

minutes and cleans the inside of the indoor

30

unit.

Using the Comfort Sleep Function

This function operates the air conditioner

automatically to make the comfortable sleep

environment.

•

,

are displayed on the display screen.

NOTE

•

This function is available with Cooling Mode.

•

The vane stops swing and sets indirect airflow

angle, after 30 minutes from start even though

the room temperature doesn’t decrease to

optimal temperature to sleep.

•

It is possible that a comfortable sleep

environment is different according to the

people.

Im

LH

00

EN

Using the Low Heating Function

This function operates the heating system to

maintain a minimum room temperature and

prevent objects in the room from freezing where

there is no permanent resident, such as a

vacation.

•

,

are displayed on the display screen.

NOTE

•

This function is available with Heating Mode.

•

During LH operation, when you press buttons

such as

,

,

MODE

,

FAN SPEED

, and

then return to Heating Mode, 30 °C high speed.

•

If

JET MODE

button is pressed during LH

operation, this function is disabled and Power

Heating Mode is operated immediately. (only

for power heating model)

•

In case an error occurred, operation can be

interrupted for the protection of the product.

•

COMFORT AIR

and

SWING

button cannot be

used while the LH function is in operation.

Using the Mosquito Away

Function

This function fans away mosquitos by generating

a high-frequency wave.

•

is displayed on the display screen.

NOTE

•

You can use this function without turning the air

conditioner on.

Using the Auto Clean Function

In the Cooling and Dehumidification Mode,

moisture is generated inside the indoor unit. This

function removes such moisture.

•

is displayed on the display screen.

NOTE

•

Some functions cannot be used while the Auto

Clean function is in operation.

•

If you turn off the power, the fan operates for

minutes and cleans the inside of the indoor

30

unit.

Using the Comfort Sleep Function

This function operates the air conditioner

automatically to make the comfortable sleep

environment.

•

,

are displayed on the display screen.

NOTE

•

This function is available with Cooling Mode.

•

The vane stops swing and sets indirect airflow

angle, after 30 minutes from start even though

the room temperature doesn’t decrease to

optimal temperature to sleep.

•

It is possible that a comfortable sleep

environment is different according to the

people.

Im

LH

00

SMART FUNCTIONS

EN

Using Smart ThinQ

Application

Installing Smart ThinQ

Search for the LG Smart ThinQ application from

the Google Play Store or Apple App Store on a

smart phone. Follow instructions to download

and install the application.

Product Registration

1

Run the

LG Smart ThinQ

application on a

smart phone.

2

Create an account and sign in.

3

Select

Register

.

4

Select

Air conditioner

.

5

Follow the instruction on the smart phone.

Wi-Fi Function

•

For appliances with the

logo

Communicate with the appliance from a smart

phone using the convenient smart features.

Using the Application

1

Select the appliance in the application and

connect it to a Wi-Fi network.

2

Select the menu on the upper right side to

access settings and features.

Firmware Update

Keep the appliance performance updated.

Smart Diagnosis™

If you use the Smart Diagnosis function, you will

be provided with the useful information such as

a correct way of using the appliance based on

the pattern of use.

Settings

Allows you to set various options on the

appliance and in the application.

NOTE

•

To verify the Wi-Fi connection, check that

Wi-Fi

icon on the control panel is lit.

•

LG Smart ThinQ is not responsible for any

network connection problems or any faults,

malfunctions, or errors caused by network

connection.

•

The machine supports 2.4 GHz Wi-Fi networks

only.

•

If the security protocol of the router is set

to

WEP

, you may fail to set up the network.

Please change it to other security protocols

(

WPA2

is recommended) and register the

product again.

•

For the router’s SSID (the name of the

wireless router that appears when searching

for Wi-Fi), please use only English alphabets

and numbers. Other characters may not be

supported.

•

If the appliance is having trouble connecting

to the Wi-Fi network, it may be too far from

the router. Purchase a Wi-Fi repeater (range

extender) to improve the Wi-Fi signal strength.

•

The Wi-Fi connection may not connect or may

be interrupted because of the home network

environment.

•

The network connection may not work properly

depending on the internet service provider.

•

The surrounding wireless environment can

make the wireless network service run slowly.

•

The application is subject to change for

appliance improvement purposes without

notice to users.

•

Functions may vary by model.

SMART FUNCTIONS

EN

Using Smart ThinQ

Application

Installing Smart ThinQ

Search for the LG Smart ThinQ application from

the Google Play Store or Apple App Store on a

smart phone. Follow instructions to download

and install the application.

Product Registration

1

Run the

LG Smart ThinQ

application on a

smart phone.

2

Create an account and sign in.

3

Select

Register

.

4

Select

Air conditioner

.

5

Follow the instruction on the smart phone.

Wi-Fi Function

•

For appliances with the

logo

Communicate with the appliance from a smart

phone using the convenient smart features.

Using the Application

1

Select the appliance in the application and

connect it to a Wi-Fi network.

2

Select the menu on the upper right side to

access settings and features.

Firmware Update

Keep the appliance performance updated.

Smart Diagnosis™

If you use the Smart Diagnosis function, you will

be provided with the useful information such as

a correct way of using the appliance based on

the pattern of use.

Settings

Allows you to set various options on the

appliance and in the application.

NOTE

•

To verify the Wi-Fi connection, check that

Wi-Fi

icon on the control panel is lit.

•

LG Smart ThinQ is not responsible for any

network connection problems or any faults,

malfunctions, or errors caused by network

connection.

•

The machine supports 2.4 GHz Wi-Fi networks

only.

•

If the security protocol of the router is set

to

WEP

, you may fail to set up the network.

Please change it to other security protocols

(

WPA2

is recommended) and register the

product again.

•

For the router’s SSID (the name of the

wireless router that appears when searching

for Wi-Fi), please use only English alphabets

and numbers. Other characters may not be

supported.

•

If the appliance is having trouble connecting

to the Wi-Fi network, it may be too far from

the router. Purchase a Wi-Fi repeater (range

extender) to improve the Wi-Fi signal strength.

•

The Wi-Fi connection may not connect or may

be interrupted because of the home network

environment.

•

The network connection may not work properly

depending on the internet service provider.

•

The surrounding wireless environment can

make the wireless network service run slowly.

•

The application is subject to change for

appliance improvement purposes without

notice to users.

•

Functions may vary by model.

EN

RF Radiation Exposure Statement

•

This equipment should be installed and

operated with minimum distance 20 cm

between the radiator and your body.

Open Source Software Notice

Information

To obtain the source code under GPL, LGPL,

MPL, and other open source licenses, that is

contained in this product, please visit http://

opensource.lge.com.

In addition to the source code, all referred

license terms, warranty disclaimers and

copyright notices are available for download.

LG Electronics will also provide open source

code to you on CD-ROM for a charge covering

the cost of performing such distribution (such as

the cost of media, shipping, and handling) upon

email request to opensource@lge.com. This

offer is valid for three (3) years from the date on

which you purchased the product.

Smart Diagnosis™ Using a Smart

Phone

•

For appliances with the

or

logo

Use this function if you need an accurate

diagnosis by an LG Electronics customer

information centre when the appliance

malfunctions or fails.

Smart Diagnosis™ can not be activated

unless the appliance is connected to power.

If the appliance is unable to turn on, then

troubleshooting must be done without using

Smart Diagnosis™.

1

Open the

LG Smart ThinQ

application on

the smart phone.

2

Select the air conditioner in dashboard.

3

Press

Start Smart Diagnosis

button.

4

Follow the instructions on the smart phone.

NOTE

•

The Smart Diagnosis™ function depends on

the local call quality.

•

If the Smart Diagnosis™ data transfer is poor

due to poor call quality, you may not receive

the best Smart Diagnosis™ service.

•

Be sure to keep ambient noise to a minimum or

the phone may not correctly receive the buzzer

beeps from the indoor unit.

EN

RF Radiation Exposure Statement

•

This equipment should be installed and

operated with minimum distance 20 cm

between the radiator and your body.

Open Source Software Notice

Information

To obtain the source code under GPL, LGPL,

MPL, and other open source licenses, that is

contained in this product, please visit http://

opensource.lge.com.

In addition to the source code, all referred

license terms, warranty disclaimers and

copyright notices are available for download.

LG Electronics will also provide open source

code to you on CD-ROM for a charge covering

the cost of performing such distribution (such as

the cost of media, shipping, and handling) upon

email request to opensource@lge.com. This

offer is valid for three (3) years from the date on

which you purchased the product.

Smart Diagnosis™ Using a Smart

Phone

•

For appliances with the

or

logo

Use this function if you need an accurate

diagnosis by an LG Electronics customer

information centre when the appliance

malfunctions or fails.

Smart Diagnosis™ can not be activated

unless the appliance is connected to power.

If the appliance is unable to turn on, then

troubleshooting must be done without using

Smart Diagnosis™.

1

Open the

LG Smart ThinQ

application on

the smart phone.

2

Select the air conditioner in dashboard.

3

Press

Start Smart Diagnosis

button.

4

Follow the instructions on the smart phone.

NOTE

•

The Smart Diagnosis™ function depends on

the local call quality.

•

If the Smart Diagnosis™ data transfer is poor

due to poor call quality, you may not receive

the best Smart Diagnosis™ service.

•

Be sure to keep ambient noise to a minimum or

the phone may not correctly receive the buzzer

beeps from the indoor unit.

MAINTENANCE

EN

WARNING

•

Before cleaning or performing maintenance, disconnect the power supply and wait until the fan

stops.

If the air conditioner will not be used for an extended period of time, dry the air conditioner to maintain

it in best condition. Clean the product regularly to maintain optimal performance and to prevent

possible breakdown.

•

Dry the air conditioner in Fan mode for 3 to 4 hours and disconnect the power.

There may be internal

damage if moisture is left in its components.

•

Before using the air conditioner again, dry the inner components of the air conditioner in Fan mode

for 3 to 4 hours. This will help to remove the odor generated from moisture.

Air Filter

•

The feature may be changed according to the type of model.

MAINTENANCE

EN

WARNING

•

Before cleaning or performing maintenance, disconnect the power supply and wait until the fan

stops.

If the air conditioner will not be used for an extended period of time, dry the air conditioner to maintain

it in best condition. Clean the product regularly to maintain optimal performance and to prevent

possible breakdown.

•

Dry the air conditioner in Fan mode for 3 to 4 hours and disconnect the power.

There may be internal

damage if moisture is left in its components.

•

Before using the air conditioner again, dry the inner components of the air conditioner in Fan mode

for 3 to 4 hours. This will help to remove the odor generated from moisture.

Air Filter

•

The feature may be changed according to the type of model.

EN

Type

Description

Interval

Air filter

Clean with a vacuum or hand wash.

2

weeks

Triple filter

Clean with a vaccum or brush.

Every 3 months

3

M filter

Clean with a vaccum or brush.

Every 6 months

Ionizer (Optional)

Use dry cotton bud to remove any dust.

Every 6 months

Indoor unit

Clean the Indoor unit surface by using a soft, dry cloth.

Regularly

Have a professional clean the condensate drain pan.

Once a year

Every 4 months

Have a professional clean the condensate drain pipe.

Replace the remote control batteries.

Once a year

Outdoor unit

Have a professional clean the heat exchanger coils and the

panel vents. (Consult with technician.)

Once a year

Have a professional clean the fan.

Once a year

Have a professional clean the condensate drain pan.

Once a year

Have a professional verify that all the fan assembly is firmly

tightened.

Once a year

Clean the electric components with air.

Once a year

NOTE

•

Never use water that is higher than 40 °C when you clean the filters. It may cause deformation or

discoloration.

•

Never use volatile substances when you clean the filters. They may damage the surface of the

product.

•

Do not wash the 3M filter with water, because the filter can be damaged (Optional).

•

Do not wash the Triple filter with water, because the filter can be damaged (Optional).

EN

Type

Description

Interval

Air filter

Clean with a vacuum or hand wash.

2

weeks

Triple filter

Clean with a vaccum or brush.

Every 3 months

3

M filter

Clean with a vaccum or brush.

Every 6 months

Ionizer (Optional)

Use dry cotton bud to remove any dust.

Every 6 months

Indoor unit

Clean the Indoor unit surface by using a soft, dry cloth.

Regularly

Have a professional clean the condensate drain pan.

Once a year

Every 4 months

Have a professional clean the condensate drain pipe.

Replace the remote control batteries.

Once a year

Outdoor unit

Have a professional clean the heat exchanger coils and the

panel vents. (Consult with technician.)

Once a year

Have a professional clean the fan.

Once a year

Have a professional clean the condensate drain pan.

Once a year

Have a professional verify that all the fan assembly is firmly

tightened.

Once a year

Clean the electric components with air.

Once a year

NOTE

•

Never use water that is higher than 40 °C when you clean the filters. It may cause deformation or

discoloration.

•

Never use volatile substances when you clean the filters. They may damage the surface of the

product.

•

Do not wash the 3M filter with water, because the filter can be damaged (Optional).

•

Do not wash the Triple filter with water, because the filter can be damaged (Optional).

EN

Clean the Air Filter

Clean the air filters once every 2 weeks, or

more, if necessary.

NOTE

•

The air filter can be broken when it is bended.

•

When the air filter is not assembled correctly,

dust and other substances come into the

indoor unit.

Type1

If you look at the indoor unit from above it, can

assemble the top filter easily.

1

Turn off the power and unplug the power

cord.

2

Hold the knobs of the air filter, lift it up

slightly.

3

Remove it from the indoor unit.

4

Clean the filter with a vacuum cleaner or with

lukewarm water with neutral detergent.

5

Dry the filter in the shade.

6

Insert the hooks of the air filter into the front

cover.

7

Push down hooks to assemble the air filter.

8

Check the side of the front cover for correct

assembly of the air filter.

Type2

1

Turn off the power and unplug the power

cord.

2

Open the front cover.

•

Lift up both sides of the cover slightly.

3

Hold the knobs of the air filters, pull them

down slightly and remove them from the

indoor unit.

4

Clean the filters with a vacuum cleaner or

with lukewarm water with neutral detergent.

5

Dry the filters in the shade.

6

Insert the hooks of the air filters into the front

cover.

7

Check the side of the front cover for correct

assembly of the air filters.

i

)

EN

Clean the Air Filter

Clean the air filters once every 2 weeks, or

more, if necessary.

NOTE

•

The air filter can be broken when it is bended.

•

When the air filter is not assembled correctly,

dust and other substances come into the

indoor unit.

Type1

If you look at the indoor unit from above it, can

assemble the top filter easily.

1

Turn off the power and unplug the power

cord.

2

Hold the knobs of the air filter, lift it up

slightly.

3

Remove it from the indoor unit.

4

Clean the filter with a vacuum cleaner or with

lukewarm water with neutral detergent.

5

Dry the filter in the shade.

6

Insert the hooks of the air filter into the front

cover.

7

Push down hooks to assemble the air filter.

8

Check the side of the front cover for correct

assembly of the air filter.

Type2

1

Turn off the power and unplug the power

cord.

2

Open the front cover.

•

Lift up both sides of the cover slightly.

3

Hold the knobs of the air filters, pull them

down slightly and remove them from the

indoor unit.

4

Clean the filters with a vacuum cleaner or

with lukewarm water with neutral detergent.

5

Dry the filters in the shade.

6

Insert the hooks of the air filters into the front

cover.

7

Check the side of the front cover for correct

assembly of the air filters.

i

)

EN

Clean the 3M and Triple Filter

(

Optional

)

1

Turn off the power and unplug the power

cord.

2

Remove the air filters from the indoor unit.

3

Remove the 3M and Triple filters from the

indoor unit.

Type1

Type2

4

Clean the filters with a vacuum cleaner.

5

Insert the 3M and Triple filters.

Type1

Type2

6

Assemble the air filters.

7

Check the side of the front cover for correct

assembly of the air filters.

NOTE

•

The feature and location of 3M and Triple filter

may vary according to the type of model.

EN

Clean the 3M and Triple Filter

(

Optional

)

1

Turn off the power and unplug the power

cord.

2

Remove the air filters from the indoor unit.

3

Remove the 3M and Triple filters from the

indoor unit.

Type1

Type2

4

Clean the filters with a vacuum cleaner.

5

Insert the 3M and Triple filters.

Type1

Type2

6

Assemble the air filters.

7

Check the side of the front cover for correct

assembly of the air filters.

NOTE

•

The feature and location of 3M and Triple filter

may vary according to the type of model.

TROUBLESHOOTING

EN

Self-diagnosis Function

This product has a built-in self-diagnosis function. If an error occurs, the lamp of the indoor unit will

blink in 2 second intervals. If this occurs, contact your local dealer or service center.

Before Calling for Service

Please check the following before you contact the service center. If the problem persists, contact your

local service center.

Problem

Possible Causes

Corrective Action

The air conditioner

does not work

normally.

Burning smell and strange

sounds are coming from the

unit.

•

Turn off the air conditioner, unplug the

power cable or disconnect the power

supply, and contact the service center.

Water leaks from the indoor

unit even when the humidity

level is low.

The power cable is damaged or

it is generating excessive heat.

A switch, a circuit breaker

(

safety, ground), or a fuse is

not operated properly.

The unit generates an error

code from its self-diagnosis.

The air conditioner

does not work.

The air conditioner is

unplugged.

•

Check whether the power cord is

plugged into the outlet or the power

isolators are switched on.

A fuse exploded, or the power

supply is blocked.

•

Replace the fuse or check if the circuit

breaker has tripped.

A power failure has occurred.

•

Turn of

f the air conditioner when a

power failure occurs.

•

When the power is restored, wait

3

minutes, and then turn on the air

conditioner.

The voltage is too high or too

low

.

•

Check if the circuit breaker has tripped.

The air conditioner was turned

off automatically at a preset

time.

•

Turn the air conditioner on.

The battery setting in the

remote controller is incorrect.

•

Make sure that the batteries are

inserted correctly in your remote

controller.

•

If the batteries are placed correctly,

but the air conditioner still does not

operate, replace the batteries and try

again.

TROUBLESHOOTING

EN

Self-diagnosis Function

This product has a built-in self-diagnosis function. If an error occurs, the lamp of the indoor unit will

blink in 2 second intervals. If this occurs, contact your local dealer or service center.

Before Calling for Service

Please check the following before you contact the service center. If the problem persists, contact your

local service center.

Problem

Possible Causes

Corrective Action

The air conditioner

does not work

normally.

Burning smell and strange

sounds are coming from the

unit.

•

Turn off the air conditioner, unplug the

power cable or disconnect the power

supply, and contact the service center.

Water leaks from the indoor

unit even when the humidity

level is low.

The power cable is damaged or

it is generating excessive heat.

A switch, a circuit breaker

(

safety, ground), or a fuse is

not operated properly.

The unit generates an error

code from its self-diagnosis.

The air conditioner

does not work.

The air conditioner is

unplugged.

•

Check whether the power cord is

plugged into the outlet or the power

isolators are switched on.

A fuse exploded, or the power

supply is blocked.

•

Replace the fuse or check if the circuit

breaker has tripped.

A power failure has occurred.

•

Turn of

f the air conditioner when a

power failure occurs.

•

When the power is restored, wait

3

minutes, and then turn on the air

conditioner.

The voltage is too high or too

low

.

•

Check if the circuit breaker has tripped.

The air conditioner was turned

off automatically at a preset

time.

•

Turn the air conditioner on.

The battery setting in the

remote controller is incorrect.

•

Make sure that the batteries are

inserted correctly in your remote

controller.

•

If the batteries are placed correctly,

but the air conditioner still does not

operate, replace the batteries and try

again.

EN

Problem

Possible Causes

Corrective Action

The air conditioner

does not emit cool

air.

Air is not circulating properly.

•

Make sure that there are no curtains,

blinds, or pieces of furniture blocking

the front of the air conditioner.

The air filter is dirty.

•

Clean the air filter once every 2 weeks.

•

See “Clean the Air Filter” for more

information.

The room temperature is too

high.

•

In summer, cooling the indoor air fully

may take some time. In this case,

select the Jet Mode to cool the indoor

air quickly.

Cold air is escaping from the

room.

•

Make sure that no cold air is escaping

through the ventilation points in the

room.

The desired temperature

is higher than the current

temperature.

•

Set the desired temperature to a level

lower than the current temperature.

There is a heating source

nearby.

•

Avoid using heat generators like

electric ovens or gas burners while the

air conditioner is in operation.

Fan Mode is selected.

•

During Fan Mode, air blows from

the air conditioner without cooling or

heating the indoor air.

•

Switch the operation mode to cooling

operation.

Outside temperature is too

high.

•

The cooling effect may not be

suf

ficient.

The fan speed

cannot be adjusted.

The Jet Mode, or Auto

Operation Mode is selected.

•

In some operation modes, you

cannot adjust the fan speed. Select

an operation mode in which you can

adjust the fan speed.

The temperature

cannot be adjusted.

The Fan Mode or Jet Mode is

selected.

•

In some operation modes, you cannot

adjust the temperature. Select an

operation mode in which you can

adjust the temperature.

The air conditioner

stops during

operation.

The air conditioner is suddenly

turned off.

•

The T

imer Function may have timed

out, which turns the unit off. Check the

timer settings.

A power failure has occurred

during operation.

•

Wait for the power to come back. If

you have the

Auto Restart function

enabled, your unit will resume its last

operation several minutes after power

is restored.

EN

Problem

Possible Causes

Corrective Action

The air conditioner

does not emit cool

air.

Air is not circulating properly.

•

Make sure that there are no curtains,

blinds, or pieces of furniture blocking

the front of the air conditioner.

The air filter is dirty.

•

Clean the air filter once every 2 weeks.

•

See “Clean the Air Filter” for more

information.

The room temperature is too

high.

•

In summer, cooling the indoor air fully

may take some time. In this case,

select the Jet Mode to cool the indoor

air quickly.

Cold air is escaping from the

room.

•

Make sure that no cold air is escaping

through the ventilation points in the

room.

The desired temperature

is higher than the current

temperature.

•

Set the desired temperature to a level

lower than the current temperature.

There is a heating source

nearby.

•

Avoid using heat generators like

electric ovens or gas burners while the

air conditioner is in operation.

Fan Mode is selected.

•

During Fan Mode, air blows from

the air conditioner without cooling or

heating the indoor air.

•

Switch the operation mode to cooling

operation.

Outside temperature is too

high.

•

The cooling effect may not be

suf

ficient.

The fan speed

cannot be adjusted.

The Jet Mode, or Auto

Operation Mode is selected.

•

In some operation modes, you

cannot adjust the fan speed. Select

an operation mode in which you can

adjust the fan speed.

The temperature

cannot be adjusted.

The Fan Mode or Jet Mode is

selected.

•

In some operation modes, you cannot

adjust the temperature. Select an

operation mode in which you can

adjust the temperature.

The air conditioner

stops during

operation.

The air conditioner is suddenly

turned off.

•

The T

imer Function may have timed

out, which turns the unit off. Check the

timer settings.

A power failure has occurred

during operation.

•

Wait for the power to come back. If

you have the

Auto Restart function

enabled, your unit will resume its last

operation several minutes after power

is restored.

EN

Problem

Possible Causes

Corrective Action

The indoor unit is

still operating even

when power has

been turned off.

The Auto Clean function is

being operated.

•

Allow the Auto Clean function to

continue since it removes any

remaining moisture inside the indoor

unit. If you do not want this feature,

you can turn the unit off.

The air outlet on

the indoor unit is

discharging mist.

The cooled air from the air

conditioner makes mist.

•

When the room temperature

decreases, this phenomenon will

disappear.

W

ater leaks from the

outdoor unit.

In the heating operations,

condensed water drops from

the heat exchanger.

•

This symptom requires installing

a drain hose under the base pan.

Contact the installer.

There is noise or

vibration.

A clicking sound can be

heard when the unit starts or

stops due to movement of the

reversing valve.

Creaking sound: The plastic

parts of the indoor unit creak

when they shrink or expand

due to sudden temperature

changes.

Flowing or Blowing sound:

This is the flow of refrigerant

through the air conditioner.

•

These are normal symptoms. The

noise will stop.

The indoor unit gives

off an odor.

Odors (such as cigarette

smoke) may be absorbed into

the indoor unit and discharged

with airflow.

•

If the smell does not disappear, you

need to wash the filter. If this does not

work, contact the service center to

clean your heat exchanger.

The air conditioner

does not emit warm

air.

When Heating Mode starts, the

vane is almost closed, and no

air comes out, even though the

outdoor unit is operating.

•

This symptom is normal. Please wait

until the unit has generated enough

warm air to blow through the indoor

unit.

The outdoor unit is in

Defrosting Mode.

•

In Heating Mode, ice/frost is built up on

the coils when the outside temperature

falls. This function removes a layer of

frost on the coil, and it should finish in

approximately 15 minutes.

Outside temperature is too low

.

•

The heating effect may not be

suf

ficient.

NOTE

•

Some functions may not be supported, depending on the model.

EN

Problem

Possible Causes

Corrective Action

The indoor unit is

still operating even

when power has

been turned off.

The Auto Clean function is

being operated.

•

Allow the Auto Clean function to

continue since it removes any

remaining moisture inside the indoor

unit. If you do not want this feature,

you can turn the unit off.

The air outlet on

the indoor unit is

discharging mist.

The cooled air from the air

conditioner makes mist.

•

When the room temperature

decreases, this phenomenon will

disappear.

W

ater leaks from the

outdoor unit.

In the heating operations,

condensed water drops from

the heat exchanger.

•

This symptom requires installing

a drain hose under the base pan.

Contact the installer.

There is noise or

vibration.

A clicking sound can be

heard when the unit starts or

stops due to movement of the

reversing valve.

Creaking sound: The plastic

parts of the indoor unit creak

when they shrink or expand

due to sudden temperature

changes.

Flowing or Blowing sound:

This is the flow of refrigerant

through the air conditioner.

•

These are normal symptoms. The

noise will stop.

The indoor unit gives

off an odor.

Odors (such as cigarette

smoke) may be absorbed into

the indoor unit and discharged

with airflow.

•

If the smell does not disappear, you

need to wash the filter. If this does not

work, contact the service center to

clean your heat exchanger.

The air conditioner

does not emit warm

air.

When Heating Mode starts, the

vane is almost closed, and no

air comes out, even though the

outdoor unit is operating.

•

This symptom is normal. Please wait

until the unit has generated enough

warm air to blow through the indoor

unit.

The outdoor unit is in

Defrosting Mode.

•

In Heating Mode, ice/frost is built up on

the coils when the outside temperature

falls. This function removes a layer of

frost on the coil, and it should finish in

approximately 15 minutes.

Outside temperature is too low

.

•

The heating effect may not be

suf

ficient.

NOTE

•

Some functions may not be supported, depending on the model.

Memo

Memo

Memo

Memo

Memo

Memo